## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Automobile Engg.

Subject: Design & Drawing of Auto Components(4067)

# Time Duration: 4 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Sketch stress strain diagram for mild steel and cast iron. Compare them. |  |
| b) | Distinguish between “Elasticity” & “Plasticity”. |  |
| c) | Explain with a neat sketch, the construction of woodruff Key. What are its advantages and disadvantages? |  |
| d) | What is the function of Couplings and Clutches? How does a coupling differ from a clutch? |  |
| e) | What do you mean by a bolt of uniform strength? |  |
| f) | State the different end conditions in a column and the effect of end conditions on the crippling load capacity of a column. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List the different types of springs and their materials. Also state what type of stresses are produced in the wire of a closed coil helical spring and draw a neat sketch showing distribution of stresses. |  |
| b) | Design a helical compression spring to carry a load of 1500 N, deflection of 40mm. Considering the space limitations, the spring index may be taken as 5. The shear stress in the spring wire should not exceed 400. |  |
| c) | Determine the width and thickness of the leaves of a six-leaf steel cantilever spring 300mm long to carry a load of 1500 N with a deflection of 30mm. The maximum stress in this spring should not exceed 320 x 106 N/m2. Take E=20 x 1010N/m2. |  |

………2/-

-2-

| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | A transmission shaft is supported on two bearings which are 1m apart. Power is supplied to the shaft by means of a flexible coupling, which is located to the left of left hand bearing. Power is transmitted from the shaft by means of a belt pulley, 250mm diameter, which is located at a distance of 300mm from the left hand bearing. The mass of the pulley is 20kg and the ratio of belt tension on tight and slack sides is 2:1. The belt tensions act vertically downward. The shaft is made of steel FeE 300 (Syt=300N/mm2) and the factor of safety is 3. Determine the shaft diameter if it transmits 10 KW power at 360 rpm from the coupling to the pulley. Ref. Fig.1. |  |
| b) | An offset link subjected to a force of 25KN is shown in Fig 2. It is made of grey cast iron FG 300 and the factor of safety is 3. Determine the dimensions of the cross section of the link. |  |
| c) | The shaft of an overhang crank is subjected to force F of 1.5KN as shown in fig (3). The shaft is made of 30 Mn2 steel having the allowable shear strength equal to 80N/mm2. Determine the diameter of the shaft. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A steel plate subjected to force of 5KN and fixed to a channel by means of three identical bolts is shown in fig (4). The bolts are made from plain carbon steel 45C8 (Syt=380N/mm2) and the factor of safety is 3. Specify the size of bolts. |  |
| b) | Design a rectangular key for a shaft 50mm diameter if the shearing and crushing stresses are limited to 42 N/mm2 and 70 N/mm2 respectively. Same material is used for shaft and key. |  |
| c) | In a rigid flanged coupling to transmit 20KW at 700 R.P.M, the flanges are made of cast iron and the other parts of mild steel. Taking ft= 28 N/mm2 for C.I fc= 60N/mm2 for C.I fs= 10N/mm2 for C.I ft= fc= 110N/mm2 for M.S fs= 40N/mm2 for M.S. fbearing = 25N/mm2 for M.S. Find i) diameter of shaft ii) diameter of bolts, assuming no. of bolts as 6 iii) flange dimensions. |  |

……..3/-

-3-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Design a bush pin type flexible coupling for connecting a motor shaft to pump shaft for the following service conditions: Power to be transmitted = 40 KW R.P.M of motor shaft = 1000 Diameter of motor shaft = 50mm Diameter of pump shaft = 45mm The bearing pressure in the rubber bush is 0.35 N/mm2 Given data: Allowable stress in the pins is 25 N/mm2 Power to be transmitted = 40KW R,P.M. of motor shaft, N = 100 Diameter of motor shaft (d) = 50mm Diameter of pump shaft = 45mm Bearing pressure in the bush (Pb)= 0.35N/mm2. Allowable stress in the pins = 25N/mm2 |  |
| b) | A multi-disc clutch employs 3 steel and 2 bronze discs having outer diameter 300 mm and inner diameter 200 mm. If the coefficient of friction is assumed as 0.22, find the axial pressure and horse power transmitted if the unit normal pressure is 0.13N/mm2 and RPM is 750. |  |
| c) | Determine the diameters of the hollow push rod made of mild steel of an I.C engine if the maximum force exerted by the push rod is 1500N. The length of the push rod is 50cms. Take factor of safety as 2.5 and the end fixity coefficient as 2. Take E=210KN/mm2. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with next sketches the different types of fits. Give their applications in brief. |  |
| b) | Design the crown thickness of a CI piston for single acting 4 stroke engine for the following specifications: Cylinder bore = 102 mm. Stroke = 120 mm. Max. gas pressure = 4.8 MPa Brake mean effective pressure = 0.6 MPa Speed = 2200 rpm. Fuel consumption = 225gm/KWh. Allowable tensile stress for C.I = 37 MPa Temperature difference between centre and edge of crown= 220 C. Heat conductivity factor = 47w/m/C. HCV of fuel = 41880 KJ/kg. |  |

…..4/-

-4-

| c) | Following are the specifications for a four –stroke petrol engine. Piston diameter = 120mm Stroke = 140mm Maximum speed = 40 rev/sec Length of connecting rod from centre to centre = 315 mm. Weight of the reciprocating parts = 18N Design the diameter of the two big end bolts and the thickness of the cap. Given that the distance between the centers of the two bolts as 62mm and length of big end bearing as 52mm. Take allowable tensile stress for both materials as 120MPa. |  |
| --- | --- | --- |