## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Theory of Structures-II(4023)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Calculate the Fixed end moments of a Fixed beam of span 4 m having a central point load of 5KN. EI =1010 mms 4. |  |
| b) | Define stiffness & relative stiffness in a beam. |  |
| c) | State any 3 assumptions made in Euler’s Column Theory. |  |
| d) | Determine slope at the free end of a cantilever, with a point load of 1.5 KN at free end & span of 1.5m. |  |
| e) | Determine the reaction of prop for a proped cantilever having a u.d.l of W KN/m run on the entire length. |  |
| f) | Draw a portal frame with central point load on horizontal member. Draw BMD taking hinged ends. (figure only) |  |
| Q.No.2. | Answer any one of the following: | (12) |
| a) | A cantilever beam of 120 mm width, 150 mm depth carries a u.d.l of 10 KN/m over its entire span of 2.4m. Determine maximum slope & deflection using moment area method. E=180 GPa. Draw the deflected shape. |  |
| b) | A SS beam, 6 m long, carries a point load of 6 KN at the centre. It has u.d.l of 10 KN/m on entire span. Calculate the slope & deflection which is maximum. Draw deflection curve. |  |
| Q.No.3. | Answer any one of the following: | (12) |
| a) | A fixed beam of 4 m span, carrying a u.d.l. of 10 KN/m over the entire span & 10 KN point load at mid span has EI =2500 KN-m2. Draw SFD & BMD. |  |
| b) | A fixed beam of 6 m span has u.d.l of 2 KN/m on left half span. Determine the FEMS & draw SFD & BMD. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A T-section is used as a strut 4 m long. The size of T section is 150mm x 120mm x 20mm and both ends of the strut are hinged. Calculate the crippling load. E =200 Gpa. |  |
| b) | A CI column of hollow cylindrical section 5m long, with ends firmly built-in, has to carry an axial load of 30t. Determine the section, using a fos of 8. Internal diameter to be 0.8 x external diameter. fc=5500 Kg/cm2 & = 1/1600. |  |
| c) | A M.S. tube 25 mm internal diameter & 32 mm external diameter, length 3 m is used as a strut, one end fixed, the other hinged. Calculate the collapsing load using E=2.1x105 N/mm2. |  |

………2/-

-2-

| Q.No.5. | Answer any one of the following: | (12) |
| --- | --- | --- |
| a) | A portal frame ABCD, AB & CD being vertical, with ends A & B fixed, carries a u.d.l of 10 KN/m on BC (i.e. the horizontal member). Analyse the portal frame & draw BMD. |  |
| b) | A beam ABCD, fixed at A & D has the following loading: 8 KN point load at 1 m from point A, on AB; a u.d.l of 5 KN/m on BC, a central point load of 4 KN on CD. l(AB)=3m, l(BC)=4m & l(CD)=4m. Analyse the beam & draw SFD & BMD. |  |
| Q.No.6. | Answer any one of the following: | (12) |
| a) | A three hinged parabolic arch ACD of span 30 m has its supports at depth 4m & 16 m below the crown hinge ‘C’. The arch carries a point load of 60 KN at a distance of 5m left of C and a point load of 120 KN at a distance of 10 m right of C. Find the reactions at the supports & BM under the load. |  |
| b) | A 3 hinged arch ACB of two quadrantal parts AC & CB of radii 2 m & 4m respectively has 2 point loads of 40 KN & 60 KN on left & right side of the crown C. The loads are at an angle of 45 with horizontal from the centre of each quadrant. Determine the support reactions & BM under the load. |  |

***********