## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Mechanical Engg.

Subject: CAD/CAM(5092)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What are the advantages to be gained by the adoption of CAD? |  |
| b) | What are the limitations found in the general wireframe modeling systems? |  |
| c) | Explain what you understand by the word ‘Numerical Control’. |  |
| d) | What are NC words? |  |
| e) | What are end effectors used in industrial robots? |  |
| f) | What is FMS? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe the functioning of a central processing unit with the aid of a block diagram. |  |
| b) | How do you classify the various geometric modeling systems based on their capabilities? Explain. |  |
| c) | What do we expect a geometric modeling system to accomplish, in a broad sense, in the total manufacturing scene? |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the advantages and disadvantages of using surface models? |  |
| b) | Explain where a NC machine is most suitable? |  |
| c) | Compare straight line and continuous path motion control system. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Refer to fig.1. Write a part programme using ISO G&M codes to turn the part shown on a CNC lathe. |  |
| b) | Refer to fig.2. Write a part programme using ISO G&M codes on a CNC machining centre. |  |
| c) | Explain the use of subroutines in part programming with an example. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the various types of motion control possible in a robot. |  |
| b) | Discuss the various material handling applications for which the robots are used. |  |
| c) | Describe the requirement of sensors in robots |  |

………2/-

-2-

| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | With a block diagram, explain the various components present in a FMS. |  |
| b) | Briefly explain the advantages that will be gained by the implementation of CIM. |  |
| c) | Write a short notes on: (any two) |  |
|  | 1.Hydraulic automation system |  |
|  | 2.DNC |  |
|  | 3.Input devices for NC machines |  |