## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Electrical Engg.

Subject: Testing & Maintenance of Electrical Equipments(5104)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What do you mean by indirect testing? State its advantages and disadvantages. |  |
| b) | The measured values of resistance at the line terminals of a star-delta connected transformer are 3 ohm & 2 ohms respectively. What will be the resistance per phase of primary and secondary windings? |  |
| c) | Give any four causes of failure of Induction motor. |  |
| d) | What is “High voltage Test” on Induction motor? What is the significance of this test? |  |
| e) | Why is sudden three-phase short circuit test performed on synchronous machines? |  |
| f) | State the various methods of revarnishing of winding of electrical equipments. |  |
| g) | State the important points to be considered regarding care of electrical equipment during period of inactivity. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat circuit diagram for back to back test on two identical d. c. machines and explain the procedure for the test. |  |
| b) | A dc shunt machine has an armature resistance of 0.5 & field resistance of 750. When run as a motor on no load at 500V applied terminal voltage, the line current was 3A. Estimate the efficiency of the machine when it operates as a generator with an output of 2 KW at 500V, the field circuit resistance remains unchanged. |  |
| c) | A 500 V DC shunt generator has a full load current of 200 A. In amative resistance is 0.1 , field resistance is 100  and constant losses including stray load losses & field copper losses is 4000 W. Calculate the efficiency i) at full load ii) at half full load. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Enlist various tests to be performed on a transformer under Dielectric testing. Explain any one in detail. |  |
| b) | Draw a circuit diagram with measuring instruments & their ranges for performing open circuit test on a 100 KVA, 11KV/400V delta star transformer. Also explain the procedure for the test. |  |
| c) | What is the significance of temperature rise test? How is it carried out on a power transformer? |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | - Justify the following statement “No load test is conducted on LV side & short circuit test is conducted on HV side of a transformer. - Explain how Insulation resistance test is conducted on a power transformer. |  |

………2/-

-2-

| b) | What are routine tests? Enlist various types of routine tests to be performed on Induction motor. Explain in brief locked rotor test on Induction motor. |  |
| --- | --- | --- |
| c) | The power input to a 500V, 50 HZ, 6 pole, three phase induction motor running at 975 rpm is 40 KW. The stator Cu losses are 2 KW. The friction, windage & core losses are 2 KW. Calculate - Slip - Rotor on losses - Power output - Efficiency. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe the following tests for Induction Motor - High Voltage Test - Resistance Measurement Test. |  |
| b) | State the various types of direct & indirect methods used for testing induction motor to determine losses & efficiency. Also state advantages & disadvantages of each method. |  |
| c) | With a neat circuit diagram explain how direct load test is performed on synchronous machine. |  |
| Q.No.6. | Write a short note on any two: | 2 x 6 = 12 |
| a) | Various methods of measurement of temperature rise of various parts of electrical machine |  |
| b) | Causes of failure in transformer and its frequency of occurrence |  |
| c) | Methods of cleaning & drying of insulation of rotating electrical machines |  |

***********