## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Theory of Structures-I(4022)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define (i) Flitched Beam (ii) Equivalent section. |  |
| b) | Define (i) limit of eccentricity (ii) Cove of section. |  |
| c) | Define statically determinate and Indeterminate structures with examples. |  |
| d) | Define Principal planes and Principal stresses. |  |
| e) | State the assumption made in the Theory of Torsion. |  |
| f) | Draw Influence diagram for simply supported beam for (i) Reactions (ii) Bending moment at a section. |  |
| g) | A rolling load of 40KN is moving on a simply supported girder of 12m span. Determine Bending moment at centre of girder using ILD concept. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A wooden beam of 125mm wide, 250mm deep consists of steel plate of 10mm thick and 125mm wide connected to top and bottom of wooden beam. If stress produced in steel is 70N/mm2, find the Moment of resistance of beam. Take Es=16Ew. |  |
| b) | A timber beam of 160mm wide and 240mm deep consists of steel plate of 10mm thick and 200mm deep placed between two wooden beam sections symmetrically. If the section is subjection to Bending Moment of 4KN-m, determine stress in steel and wood. Take Es=20Ew. |  |
| c) | Find the degree of indeterminacy of following structures: (i) A Fixed Beam (ii) Two hinged Arch (iii) King Post Truss. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A rectangular column of 300mm wide and 500mm deep carries a vertical load of 600KN on YY axis at an eccentricity of 150mm from XX axis. Determine the maximum and minimum stresses produced at base of column. |  |
| b) | A Bridge peir of 1.5m wide, 2.0m deep carries an eccentric load of 1000KN at a distance of 300mm below X-axis and 400mm to the left of Y-axis. Determine maximum and minimum stresses at all corers of peir. |  |
| c) | A hollow circular column has outer diameter 200mm and internal diameter 150mm. It carries an axial load of 60KN and eccentric load of 90KN at an eccentricity of 80mm from the axis. Find maximum and minimum stresses due to both loads. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | At a certain point in a strained material, normal stresses of 100N/mm2 tensile and 40N/mm2 compressive acts. Find resultant stress across a plane making an angle of 50 to major stress using Mohr’s circle method. |  |

………2/-

-2-

| b) | A shaft is subjected to normal stresses of 80N/mm2 and 40N/mm2 both tensile in two mutually perpendicular directions. Also a shear stress of 35N/mm2 acts across both planes. Determine the magnitude and direction of Principal stresses and maximum shear stresses. |  |
| --- | --- | --- |
| c) | At a point in a Boiler shell, principal stresses are 100N/mm2 and 60N/mm2 both tensile. Also a shear stress of 30N/mm2 acts across both planes. Determine Normal stress, Tangential stress and Resultant stress on a plane at 30 with minor stress. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A simply supported beam of 8m span carries point loads of 20KN, 40KN and 60KN at a distances of 2m between them. Determine reaction at supports using Influence Line Diagram. |  |
| b) | Two wheel loads of 40KN and 60KN spaced at 1.4m apart with 60KN leading load is moving on the girder of span 12m. Determine maximum bending at a section of 5m from left end of girder. Use ILD concept. |  |
| c) | A simply supported girder has 16m span. A live load of 40KN/m of length 4m is moving on the girder from left to right. Determine Absolute maximum bending moment at centre of girder. Use ILD concept. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A solid shaft is to transmit a torque of 26KN-m. If the maximum shear induced in the shaft is not to exceed 65N/mm2, determine the diameter of shaft. Also find angle of twist in a length of 2m shaft. Take Regidity Modulus G=80x103N/mm2. |  |
| b) | A shaft used in Flange coupling is subjected to torsional moment of 4x106 N-mm and a Bending Moment of 2.5x106 N-mm. Determine diameter of shaft, if maximum Principal stress is 80N/mm2. Also find maximum shear stress in shaft. |  |
| c) | A solid circular shaft of 60mm diameter is rotating at 100 rpm. If the maximum shear stress in shaft material is 50N/mm2, determine (i) Maximum Torque (ii) Mean Torque (iii) Power Transmitted. Take maximum torque exceeds mean torque by 45. |  |

***********