## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Civil Engg./CT/FT&EE

Subject: Design of Structures-I(Steel)[4024]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of IS 800-1984 code & steel tables is permissible.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Draw labelled sketch of Fillet weld and state salient features. |  |
| b) | State the advantages and disadvantages of welded joints. |  |
| c) | Distinguish between Web Buckling and Web Crippling in beams. |  |
| d) | State merits and demerits of tubular sections. |  |
| e) | With the aid of neat sketches explain how the lateral restraint is provided to steel beams. |  |
| f) | With a neat sketch state the components of Plate girder and their applications. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A tie member ISMC 225 @ 254.1 N/m is connected to a gusset plate by fillet welds. Design the connection for the full strength of channel such that overlap of web of channel with gusset plate is limited to 275 mm. Use slots if required. Take stress in weld as 108 MPa and tensile stress=150 MPa. |  |
| b) | Design a lap joint to connect two plates 130x10mm and 130x8mm by fillet welds. Take tensile stress 150N/mm2 and shear stress in weld 110MPa. |  |
| c) | Design a fillet weld joint for connecting two angle sections ISA 100x100x10mm on either side of gusset plate to carry a load of 180 KN. Use shear stress in weld as 108 MPa. |  |
| Q.No.3. | Answer the following: |  |
| a) a) | A tie member consists of two ISA 100x65x8mm. Find its tensile strength in following cases: - Angles connected back to back by its longer leg without tack welds. - Angles connected to same side of gusset plate by its longer leg with tack welds. ### OR Design a tension member subjected to a pull of 180KN using unequal angle section. Length of member is 2.5 m. Also design welds for member. Take shear stress=108 MPa for welds. | (08) (08) |
| b) | Determine the tensile strength of a roof truss tie member ISA 150x115x10mm connected to gusset plate by 6mm fillet welds. | (04) |

………2/-

-2-

| Q.No.4. | Answer the following: |  |
| --- | --- | --- |
| a) | Briefly explain the procedure of design of Slab Base. | (6) |
| b) | Design a column section using ISHB steel section to carry a load of 650 KN. The length of column is 6m with one end fixed & other hinged. | (6) |
|  | #### OR |  |
| Q.No.4. | Answer the following: |  |
| a) | Design a column to support an axial load of 1400 KN using two channel sections placed with its web back to back. The effective height of column is 6.5 m. | (10) |
| b) | State the importance of Slenderness ratio in compression members. | (2) |
| Q.No.5. | Answer the following: |  |
| a) | Design a simply supported beam to carry an uniformly distributed load of 30 KN/m over an effective span of 6m. Check the section for shear, deflection, web buckling and web crippling. | (10) |
| b) | State the situations in which Built up beams are used with neat sketches. | (2) |
|  | #### OR |  |
| Q.No.5. | Design a Gusseted Base for a Built up column ISHB 300 @ 618 N/m along with cover plates of 300x12mm connected to both flanges of column to carry a load of 1300 KN. Use bearing stress of concrete as 4 N/mm2. Also design welds and bolts for connection. | (12) |
| Q.No.6. | Answer the following: |  |
| a) | Draw line diagram of various types of roof trusses stating their use. | (6) |
| b) | Design the angle purlin for the following data: centre to centre of trusses = 4m Length of panel = 1.45 m Wt of AC sheet roof coverings = 160 N/m2 Wind normal to roof = 1000N/m2 Slope of roof = 28º. | (6) |
|  | #### OR |  |
| Q.No.6. | The sliding end of roof truss rests on 230mm thick masonry wall through a concrete pad. Maximum normal reaction on bearing pad is 80KN. The principal rafter is inclined at 28º to the main tie, which is horizontal. The length of panel of rafter is 1.65m. Design (i) principal rafter (ii) main tie. Use two angle sections with suitable welds. | (12) |

***********