## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Shipbuilding Engg.

Subject: Elementary Ship Design(5331)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) Q.No.1 is compulsory. Answer any five from the remaining

Questions.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Explain any five of the following: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Design features of container ship |  |
| b) | Constraints on ship dimensions due to trading route |  |
| c) | Concept design |  |
| d) | Deadweight Co-efficient |  |
| e) | Parallel middle body |  |
| f) | Tonnage Certificate |  |
| g) | Safety Plan |  |
| Q.No.2. | Answer the following: | 2 x 6 = 12 |
| a) | Explain in detail the various ship owner’s requirements stated in ship design. |  |
| b) | How and when is detail design carried out in ship design? Explain in context with design spiral. |  |
| Q.No.3. | Estimate the main dimensions, deadweight, displacement of a 25m long fishing trawler with endurance of 14 days and for a catch of 15 tonnes. The total power of the vessel is 450HP. The fuel consumption is at the rate of 155 gms/BHP/hour. The oil consumption is 3gms/BHP/hour. For the stipulated endurance, provision required is 0.15 tonne/person, fresh water required is 1 tonne/person, crew and effects is 0.15 tonne/person. Total number of crew is 20. ### Given that dwtΔ=0.22, CB=0.5−0.54 LB=4.2 to 4.5, BT=2.5 to 2.68, TD=0.7. | (12) |
| Q.No.4. | Answer the following: |  |
| a) | Explain how the machinery weight of a vessel can be estimated in the preliminary design. | (04) |
| b) | A ship in loaded condition has a displacement of 16420 tonnes, draft 7.4m, CB=0.66, CW=0.767. If the length of the ship is =120m, estimate the TPC and MCT at this loaded condition. Also estimate the draft for a displacement of 8800 tonnes. | (08) |
| Q.No.5. | Answer the following: |  |
| a) | Discuss in brief ‘Lines Plan’ for a ship. | (04) |
| b) | How sectional area curve can be obtained? | (08) |

………2/-

-2-

| Q.No.6. | Draw a typical general arrangement plan of a cargo vessel. Explain the layout of the various machinery spaces and fittings on board the vessel. | (12) |
| --- | --- | --- |
| Q.No.7. | Answer the following: |  |
| a) a) | Explain specifications and contract of a ship. #### OR What constitutes a safety plan? | (06) (06) |
| b) b) | How are the freeboard calculations of a ship carried out? #### OR Explain the influence of Gross tonnage and Net tonnage on the design of a vessel. | (06) (06) |

***********