## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Electrical Engg.

Subject: Energy Management(5108)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.I. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| i) | State any three evidences on climatic change. |  |
| ii) | State the effects of poor quality of incoming power supply on performance of machinery, equipment and lighting. |  |
| iii) | What is the need for flattening of load curve? |  |
| iv) | Explain the need for energy audit. |  |
| v) | Nameplate details of motor are given as Power =15KW, efficiency = 0.9. Actual power drawn = 8 KW. Find the percentage loading of the motor. |  |
| vi) | Draw and explain the system resistance curve for pumps. |  |
| vii) | List down the preparations to be done before the assessment of performance of lighting. |  |
| Q.No.II. | Answer any two of the following Questions: | 2 x 6 = 12 |
| i) | Briefly explain any six features of Energy conservation Act 2001. |  |
| ii) | Write a short note on benefits of Energy conservation for industry, nation and globe. |  |
| iii) | Write a short note on Energy Audit measuring instruments. |  |
| Q.NoIII. | Answer any two of the following Questions: | 2 x 6 = 12 |
| i) | Explain step by step approach for M D control. |  |
| ii) | List down the components of electrical tariff system structure. |  |
| iii) | Briefly explain the concept of maximum demand. In an industry, the drawl over a recording cycle of 30 minutes is 2500 KVA for 4 minutes 3600 KVA for 12 minutes 4100 KVA for 6 minutes 3800 KVA for 8 minutes Determine the maximum demand. |  |
| Q.No.IV. | Answer any two of the following Questions: | 2 x 6 = 12 |
| i) | Write a short note on location of capacitors. |  |
| ii) | Briefly explain the concept of p.f. improvement. The utility bill shown an average p.f. of 0.68 with an average KW of 900. How much KVAR capacitors are required to improve the p.f. to 0.99? |  |
| iii) | An engineering industry has installed a 1600 KVA transformer. The plant demand was 1200 KVA with p.f. of 0.72. Determine the percentage loading of transformer. Also determine the capacitor sizing to improve the p.f. to 0.95. |  |

………2/-

-2-

| Q.No.V. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| i) | Following test data are recorded for the motor: Rated power = 34 KW/45HP Voltage = 415 V Current = 57 A Speed = 1475 rpm |  |
|  | ### N L Test Data |  |
|  | V = 415 V I = 16.1 A F = 50 Hz Resistance at 30ºC = 0.264 |  |
|  | PNL = 1063.74 watts Determine the efficiency of motor at FL. |  |
| ii) | Explain the different methods to determine the flow for estimating the efficiency of pump. |  |
| iii) | Explain the procedure for estimating the efficiency of pump. |  |
| Q.No.VI | Answer any two of the following Questions: | 2 x 6 = 12 |
| i) | Write a short note on automatic power factor controller. |  |
| ii) | Write a short note on soft starters. |  |
| iii) | Explain the different watt loss areas of motor and modifications required for improving efficiency of motor. |  |

***********