## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April,2013 Examinations

Programme: Electronics Engg./EC

Subject: Data Communication(5138)

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Describe the function of the altitude control circuit of a satellite. |  |
| b) | Describe station keeping in satellite communication. |  |
| c) | What is numerical aperture of an optical fiber? |  |
| d) | Differentiate between step index and graded index optical fibers. |  |
| e) | Describe in brief the Bisync protocol. |  |
| f) | Describe circuit switching used in data communication. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat block diagram, describe the transmitter of an earth station. |  |
| b) | Describe a transponder of a communication satellite. |  |
| c) | Describe the CDMA technique used in satellite communication. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe the construction of a PIN diode detector and draw and explain its characteristic. |  |
| b) | With the help of a neat diagram, describe the structure of an optical fiber and explain how light propagates through it. |  |
| c) | What is splicing of an optical fiber and why is it required? Describe the splicing procedure. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe the transfer modes of the HDLC protocol and the I-frame format. |  |
| b) | Explain the functions of the network and transport layers of the OSI model for communication. |  |
| c) | Explain how a communication LAN is implemented using Ethernet. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe any one type of modem in detail. |  |
| b) | Describe with the help of diagrams, the following network topologies: (i)Ring (ii)Bus. |  |
| c) | Describe six advantages of optical communication. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | TDMA technique |  |
| b) | INMARSAT |  |
| c) | Losses in optical fiber communication |  |
| d) | Give two differences each, between: - Synchronous and asynchronous communication - Simplex and duplex communication. |  |

***********