## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Electrical Engg./EI

Subject: Circuit Theory(4104)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is single energy transient & double energy transient? How are transient disturbances classified? |  |
| b) | Derive the expression for power consumed in a purely resistive circuit. Draw the waveforms of voltage, current and power consumed. |  |
| c) | Find the voltage Vxy for the circuit shown in fig.1(a). |  |
| d) | State and explain Maximum power transfer theorem. |  |
| e) | With the help of a power triangle explain different powers in series RL circuit using AC supply. |  |
| f) | Define Bandwidth,  factor. Give their expressions. |  |
| g) | Draw Hay’s bridge circuit. Give the equation at balance and list the advantages and disadvantages. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Apply loop current method to find the loop currents I1, I2, I3 shown in fig.2(a). |  |
| b) | By using delta/star transformation find the current I supplied by the battery for the network shown in fig.2(b). |  |
| c) | Using Norton’s theorem find the current that would flow through the resistor R2 when it takes a value of 12. Refer fig.2c. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | When a voltage of 100V at 50Hz is applied to a choking coil A, the current taken is 8A and the power is 120W. When applied to a coil B, the current is 10A and the power is 500W. What current and power will be taken when 100V is applied to the two coils connected in series? |  |
| b) | A coil having a resistance of 15 ohm and an inductance of 0.2H is connected in series with another coil having a resistance of 25 ohm and an inductance of 0.04H to a 230V, 50Hz supply. Determine i) the voltage across each coil ii) power dissipated in each coil iii) Power factor of the circuit as a whole. |  |
| c) | Two impedances given by Zn = 10 + j5 and Z2 = 8 + j6 are joined in parallel and connected across a voltage of V = 200+j0. Calculate the circuit current, its phase and the branch currents. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Derive an expression for Resonating frequency in parallel circuit and using Graphical representation show variation of Inductive susceptance, capacitive susceptance and net susceptance with frequency. |  |

………2/-

-2-

| b) | A coil having an inductance of 50 mH and resistance 10 ohm is connected in series with a 25 F capacitor across a 200V ac supply. Calculate i) resonance frequency of the circuit ii) Current flowing at resonance and iii) value of Q0. |  |
| --- | --- | --- |
| c) | The arms of an ac Maxwell bridge are arranged as follows. AB is a non inductive resistance of 1000 in parallel with a capacitor of capacitance 0.5F. BC is a non inductive resistance of 600 ohm. CD is an inductive impedance (unknown) and DA is a non inductive resistance of 400 ohm. If balance is obtained under these conditions, find the value of resistance and the inductance of the branch CD. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Derive a relationship between line and phase quantities in a star connected system. Also give an expression for power in terms of line values. |  |
| b) | A balanced star connected load of (8+j6)  per phase is connected to a balanced 3 phase 400 volt supply. Find the line current, power factor, power and total volt amperes. |  |
| c) | Three identical impedances are connected in delta to a 3 supply of 400V. The line current is 35A and the total power taken from the supply is 15KW. Calculate the resistance and reactance values of each impedance. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Find the positive, negative and zero phase sequence components of the following set of three unbalanced voltage vectors: VA =1030 VB =30-60 VC = 15145. |  |
| b) | Three impedance coils, each having a resistance of 20 and a reactance of 15 are connected in a star to a 400V, 3, 50Hz supply. Calculate (i) the line current (ii) Power supplied and (iii) the power factor. |  |
| c) | Impedances Z2 and Z3 in parallel are in series with an impedance Z1 = (6.25 + j1.25) ohm; Z2 = (5 +j0) ohm and Z3 = (5 – jxc) ohm. Determine the value of capacitance xc such that the total current of the circuit will be in phase with the total voltage. Calculate the circuit current and power under this condition. |  |