## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Electrical Engg./E&EE

Subject: Transformers(4106)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | For transformer, define - L.V.Winding - Step down transformer. |  |
| b) | Write emf equation of the transformer. Also define voltage ratio. |  |
| c) | State any three advantages of computing quantities in per unit. |  |
| d) | Draw a neatly labelled phasor diagram of transformer on ‘No Load’. |  |
| e) | List the essential conditions for connecting two transformers in parallel. |  |
| f) | What is an autotransformer? How it is different from conventional transformer? |  |
| Q.No.2. | Answer any two of the following: | 2 x 6 = 12 |
| a) | With neat diagrams distinguish between core type and shell type transformers. |  |
| b) | With a neat diagram explain construction and working of On Load Tap changer. |  |
| c) | Briefly explain importance of following in transformer: - Conservator - Buchholz Relay. |  |
| Q.No.3. | Answer any two of the following: | 2 x 6 = 12 |
| a) | With a neat diagram explain phasor diagram of a transformer when it is supplying energy to an inductive load. |  |
| b) | A single phase transformer has 1000 turns on primary and 200 turns on secondary. The no load current is 3 amperes at a power factor of 0.2 lagging. Calculate the primary current and its power factor when secondary current is 250 amperes at a p.f. of 0.7 lagging. |  |
| c) | A 30KVA, 2400/120 V, 50 Hz transformer has a high voltage winding resistance of 0.1 ohm & leakage reactance at 0.22 ohm. The low voltage winding resistance is 0.035 ohm and leakage resistance is 0.012 ohm. Find the equivalent winding resistance, reactance and impedance referred to - H.V. side - L.V. side. |  |
| Q.No.4. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Using the vector diagram deduce the expression for approximate voltage drop in a transformer for lagging p. f. load. Also write expression for percentage voltage regulation. |  |

………2/-

-2-

| b) | A100 KVA, single phase transformer has 400 turns on the primary & 80 turns on secondary. The primary and secondary resistances are 0.3 ohm & 0.01 ohm respectively. Corresponding leakage reactance are 1.1 ohm & 0.035 ohm. The supply voltage is 2200 volt. Calculate - % voltage regulation for full load & p.f. of 0.8 lagging. - % voltage regulation for 50% full load & pf of 0.9 leading. |  |
| --- | --- | --- |
| c) | Find “all day” efficiency of a transformer having maximum efficiency of 98% at 15KVA & units p.f. the loading is as given below: - 12 hours – 2 KW at 0.5 pf lag - 6 hours – 12 KW at 0.8 p.f lag - 6 hours – at no load. |  |
| Q.No.5. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Draw and explain connection diagram & vector diagrams for following transformer: - Yy 6 - Yd 11. |  |
| b) | Two single phase transformers with equal turns have impedances of (0.5 + j3) ohm and (0.6 + j10) ohm with respect to the secondary. If they operate in parallel, determine how they will share a total load of 100 KW at 0.8 p.f. lagging? |  |
| c) | A 150 KVA transformer has full load copper loss of 1600W and iron loss of 1.4 KW. Find the efficiency of transformer at - 50% of full load at pf of 0.8 lagging - 25% of full load at pf of 0.8 leading. |  |
| Q.No.6. | Answer any two of the following: | 2 x 6 = 12 |
| a) | With a neat diagram explain principle of working of a transformer. |  |
| b) | Write a short note on ‘Welding transformer’. |  |
| c) | Write a short note on ‘High frequency transformer’. |  |

***********