## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Automobile Engg.

Subject: Automobile Chassis-II(4062)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is tractive resistance? |  |
| b) | What is traction? |  |
| c) | With diagram explain a shock absorber. |  |
| d) | Distinguish between frameless and conventional frame type vehicle body. |  |
| e) | Explain four wheel steering. |  |
| f) | Explain semi–automatic gearbox. |  |
| g) | What is a trailing link? |  |
| Q.No.2. | Answer any two of the following: | 2 x 6 = 12 |
| a) | A vehicle has weight of 10000N. Its resistance is given by the equation R=aw+bAV2 where ‘a’ is 0.002 and ‘b’ is 0.00018. Total frontal projected area is 3.2m2. V is in m/sec. Engine is developing 40000KW of power at 6000rpm. Top gear ratio is 2.1:1. Tyre diameter is 80cm. Calculate the resistance on a levelled road. |  |
| b) | Draw a road performance curve and explain the term economical speed. |  |
| c) | Explain the term ‘gradiability’. How gradiability is calculated? |  |
| Q.No.3. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Explain limiting overturning angle on a slope. |  |
| b) | Draw a Wind pressure distribution diagram and explain its necessity. |  |
| c) | How front-end safety is achieved? |  |
| Q.No.4. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Explain working of an epicyclic gear train. |  |
| b) | Explain an overdrive. |  |
| c) | Explain the working of a Variator. |  |
| Q.No.5. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Draw diagram of a telescopic shock absorber and explain its working. |  |
| b) | Explain working of a wishbone type of independent suspension. |  |
| c) | Explain the working of an Ackerman steering mechanism. |  |
| Q.No.6. | Answer any two of the following: | 2 x 6 = 12 |
| a) | A car having wheel base of 2.44m and pivot centre of 1.12m, has a track of 1.22cm between centres of tyre thread. If the angle of lock is 30º, determine the radius of outer turning circle. |  |
| b) | Explain working of a tie rod rack and pinion type of steering mechanism. |  |
| c) | How does an electronically controlled electric motor power steering works? |  |

***********