## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Electrical Engg./E&EE

Subject: Utilisation of Electrical Energy(4111)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain the different modes of heat transfer. |  |
| b) | What are the essential properties of resistance heating element? |  |
| c) | What are the advantages and applications of dielectric heating? |  |
| d) | What do you mean by arc blow? Explain the methods for reducing it. |  |
| e) | What are the different current controlling devices of resistance welding set? Explain any one. |  |
| f) | Compare the different systems of track electrification. |  |
| g) | What are the qualities of good lighting? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A cubic water tank has surface area of 6.0m2 and is filled to 90% capacity six times daily. The water is heated from 20c to 65c. The losses per square metre of tank surface per 1c temperature difference is 6.3 watts. Find the loading in KW and the efficiency of the tank. Assume specific heat of water as 4200 J/Kg/c and one KWH = 3.6MJ. |  |
| b) | What are the different methods of heat control in resistance heating? Explain any one method in detail. |  |
| c) | Explain infrared heating. What are its advantages and applications? |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Compare a.c. and d.c. arc welding. |  |
| b) | What are the advantages of three phase welding transformer over single phase welding transformer? Draw a neat circuit diagram of a three phase welding transformer set and explain the function of each part. |  |
| c) | - Describe in brief the advantages of using coated electrodes. - List down atleast 6 advantages of electric heating. | (3) (3) |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw speed-time curves of electric trains for (i) Main line service and (ii) suburban service and explain their characteristics. |  |
| b) | What are the general features of traction motors? Explain series parallel starting of DC traction motor using two motors. |  |
| c) | - Write a short note on Pantograph collector. - Explain in brief the different methods of Electric heating. | (3) (3) |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Describe with the help of a neat diagram the construction and working of a high pressure mercury vapour lamp. |  |
| b) | Explain the stroboscopic effect and the methods to eliminate this effect. |  |
| c) | Explain the terms Reflector factor, absorption factor, Maintenance factor and utilization factor. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | - Why are starter, condenser and choke used in fluorescent lamp? - Explain the advantages of using inert gas in filament lamps. | (3) (3) |
| b) | What are the various types of induction furnaces? Explain any one in detail. |  |
| c) | - Discuss in brief the relative merits and demerits of direct and indirect electric arc furnaces. - Explain the basic difference between resistance welding and arc welding. | (3) (3) |

***********