## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Shipbuilding Engg.

Subject: Diesel Engines Operation & Maintenance(5045)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Why are engine valves sometimes coated with Aluminium? |  |
| b) | How will you check the end clearance of a piston ring? |  |
| c) | What is the function of expansion reservoir in the engine cooling system? |  |
| d) | Draw a neat sketch of the valve timing diagram of a two-stroke diesel engine. |  |
| e) | Briefly explain the use of piston-ring compressor tool. |  |
| f) | What care should be taken while tightening the screws of a cylinder head? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Distinguish between two-stroke and four-stroke diesel engines. |  |
| b) | Define (i) Piston displacement (ii) Clearance Volume (iii) Compression ratio of a diesel engine. |  |
| c) | With the help of neat sketches, describe the different types of torque wrenches. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain in brief various types of turbochargers. |  |
| b) | Explain the “jet shaker effect” of oil cooled pistons. |  |
| c) | With the help of a neat sketch, explain the working of a typical Pressure-Time (P-T) fuel injection system of a diesel engine. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What precautions should be taken before stopping or shutting down a diesel engine? |  |
| b) | Describe the step–by-step procedure of setting valve clearance of a 6-cylinder diesel engine. |  |
| c) | With the help of neat sketches, explain the importance of the helical groove on the plunger of a jerk pump type fuel injection pump. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw up a typical preventive maintenance schedule for the cooling system of the diesel engine of a generator set, showing the A, B, C, D and annual checks. |  |
| b) | Prepare a trouble-shooting chart for any one of the following faults/complaints: (i) Low power or loss of power from the diesel engine (ii) Excessive lubricating oil consumption. |  |
| c) | Distinguish between preventive maintenance and breakdown maintenance? What is predictive maintenance? |  |

………2/-

-2-

| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Describe the procedure of dismantling crankshaft from a diesel engine. How is it inspected? What is the measuring tool used? How is the decision of crankshaft grinding taken? Describe the procedure of re-assembling the crankshaft to the engine. |  |
| b) | How are inlet and exhaust valves tested for leakage? Explain the procedure of reconditioning of valve, valve seats and valves guides. Name the tools involved. |  |
| c) | Explain how piston-connecting rod assembly is removed from the cylinder. How are piston rings inspected for wear? During engine overhauling how is the piston-connecting rod assembly fitted back into the cylinder. What are the various tools and gauges used? |  |

***********