## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Offshore Structures(5339)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) Q.No.1 is  compulsory. Attempt any four from the remaining

Questions.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Why do we study Oceanography? Explain. |  |
| b) | What are the different resources available from the ocean? |  |
| c) | What is Archimedes principle? |  |
| d) | Explain the terms Buoyancy, Metacentre and Centre of Buoyancy. |  |
| e) | What is pre-stressed concrete? Explain. |  |
| f) | Explain the different coatings and corrosion protection measures. |  |
| g) | What are the different Non-destructive testing methods for welds? |  |
| h) | What are the different loads acting on offshore platform? |  |
| Q.No.2. | Answer any two of the following Questions: | 2x7.5=15 |
| a) | Explain the terms Wave height, Wave period, Wave direction and design Wave height with the help of a neat diagram. |  |
| b) | Enlist the different geophysical methods used for study of the seabed. Explain any two in detail. |  |
| c) | Explain the variation of currents with depth. |  |
| Q.No.3. | Answer any two of the following Questions: | 2x7.5=15 |
| a) | What are drilling rigs? Explain Jack-ups. Draw neat sketch. |  |
| b) | What are Semi- submersibles? Explain with neat sketch. |  |
| c) | What are production platforms? Explain the jacket type with a neat sketch. |  |
| Q.No.4. | Answer any two of the following Questions: | 2x7.5=15 |
| a) | Enlist the different submarine pipeline installation methods. Explain the reel-barge method. |  |
| b) | Explain any three types of trenching methods. |  |
| c) | Enlist the different Tow methods. Explain any three. |  |
| Q.No.5. | Answer the following Questions: |  |
| a) | A uniform rectangular body 3 m long, 1m wide and 0.8 m deep floats in water, the depth of immersion being 0.6m. What is the weight of the body? Find also the position of the metacentre. | (8) |
| b) | Explain the different types of equilibrium of floating bodies. | (7) |
| Q.No.6. | Answer the following Questions: |  |
| a) | Enlist the different types of barges. Explain any three types. | (8) |
| b) | Where do we use underwater concrete and grouting? Explain the Tremie concrete method. | (7) |
|  | ### OR |  |
| b) | What are the different concrete mixes used for offshore structures? Explain in brief the different admixtures used. | (7) |

***********