## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Mechanical Engg./P

Subject: Machine Design Practice(4045)

# Time Duration: 4 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| i) | Define “factor of safety”. What are the factors affecting selection of factor of safety in Design? |  |
| ii) | Explain the following terms in connection with stress-stain diagram: a) Yield Point Stress b) Ultimate Tensile Stress c) Proof Stress d) % Elongation in length. |  |
| iii) | Why is the taper provided on a cotter? A single taper cotter is preferred over double taper cotter. Why? |  |
| iv) | A solid circular shaft needs to be designed to transmit a torque of 50 N-m. If the allowable shear stress of the material is 140MPa, assuming a factor of safety of 2, find the minimum allowable design diameter of the shaft. |  |
| v) | How are keys classified? Draw neat sketches of different types of keys and their applications. |  |
| vi) | What do you understand by “Bolts of uniform strength”? Explain with suitable sketches. |  |
| vii) | Explain any two methods of mounting of bearings on a shaft. |  |
| viii) | Write short note on “nipping” of leaf springs. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| i) | Two rods are connected by means of a knuckle joint. The axial force P acting on the rods is 25KN. The rods and the pin are made of plain carbon steel 45C8 (Syt= 380 N/mm2) and the factor of safety is 2.5. The yield strength in shear is 50% of the yield strength in tension. Calculate the a) Diameter of the rods b) outside diameter of single and double eye C) Diameter of the pin. |  |
| ii) | A cast iron pulley transmits 20KN at 300 rpm. The diameter of the pulley is 550mm and has tour straight arms of elliptical cross section in which major axis is twice the minor axis. Find the dimensions of the arm, if the allowable bending stress is 15 MPa. | …….2/- |

-2-

| iii) | Design a bell crank lever used to lift a load of 2500N acting on a pin at A as shown in the fig.1(iii). The effort is applied at the other end. The cross section of the lever is rectangular with b/t ratio as 2.5:1. Safe tensile stress t = 75N/mm2 and safe shear stress for pin material is 50N/mm2. Take bearing pressure for pin as 10 N/mm2. (take r/d ratio as 1.25:1) |  |
| --- | --- | --- |
| Q.No.3. | Answer the following Questions: |  |
| i) | - Define equivalent twisting moment and equivalent Bending moment pertaining to shaft design. | (3) |
|  | b) A line shaft is to transmit 30 KN at 160 rpm. It is driven by a motor placed directly under it by means of a belt running on the 1000 mm diameter pulley keyed to the end of the shaft. The tension on tight side of the belt is 2.51 times the tension on the slack side and the centre of the pulley overhangs at 150 mm beyond the centre line of the bearing. Determine the diameter of the shaft if allowable shear stress is 56 N/mm2 and the pulley weighs 1.6 KN. Refer fig.3(i). | (6) |
| ii) | A C-frame is subjected to a force of 15KN is shown in the fig.3(ii). It is made of grey cast iron FG 300 and the factor of safety is 3. Determine the dimension of the cross section of the frame. | (3) |
|  | ### OR |  |
| ii) | Find the minimum size of a hole that can be punched in a 20mm thick M.S. plate having an ultimate shear strength of 300MPa. The maximum permissible compressive stress in the punch material is 1200 N/mm2. | (3) |
| Q.No.4. | Answer the following Questions: | 2 x 6 = 12 |
| i) | a)A 12.5 mm plate 50mm wide carries a static tensile load of 36KN as it is to be welded to a plate by parallel fillet welds as shown in the fig.4(i). Determine the length of each weld. |  |
|  | b)A steel bar 10 x 50 mm is cantilevered with two M12 bolts (P and Q) to support a static load of 4KN as shown in fig.4(i)(b). Find the primary and secondary load on bolt P. |  |
| ii) | An ISA 200 x 100 x 10 angle is welded to a steel plate by means of filled weld as shown in fig.4(ii). The angle is subjected to a static force of 150 KN. The permissible shear stress for the weld material is 70 MPa. Design the joint if the size of the weld is 10mm. |  |
|  |  | ……3/- |
|  | -3- |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| i) | Design a cast iron rigid flange coupling to transmit 15KW at 900rpm from electric motor to a compressor. The service factor may be assured as 1.35. The following permissible stresses may be used: Shear stress for shaft, bolt and key material = 40N/mm2, crusting stress for bolt and key = 80 N/mm2 and shear stress for cast iron = 8 N/mm2. |  |
| ii) | A doubled threaded power screw with square threads is used to raise a load of 300 KN. The nominal diameter is 100 mm and the thread pitch is 12mm. The coefficient of friction of the screw threads is 0.15. Neglecting collar friction, calculate - Torque required to raise the load - Torque required to lower the load - Efficiency of the screw. |  |
| iii) | - Write down the steps involved in the design of rectangular key for a coupling - What are flexible couplings? Draw a neat sketch of any one type of flexible coupling. | (3) (3) |
| Q.No.6 | Answer any two of the following | 2 x 6 = 12 |
| i) | Design a spring for a balance to measure 0 to 1000N over a scale of 80 mm. The spring is to be enclosed in a casing of 25 mm diameter. The approximate number of turns is 30. The modulus of rigidity is 85 KN/mm2. Also calculate the maximum shear stress induced. |  |
| ii) | A semi elliptic spring used for automobile suspension consists of 2 extra full length leaves and eight graduated leaves including the master leaf. The centre to centre distance between the two eyes is 1m. The stress in the material of the spring is 1500 N/mm2, E = 2.07 x 105 N/mm2 and the factor of safety is 2. The maximum spring load is 30 KN. The leaves are pre-stressed so as to equalize stresses on all leaves under maximum load. Determine the cross section of the leaves and the deflection at the end of the spring. |  |
| iii) | - Explain how a bearing is selected from the manufacturer’s catalogue (Handbook). - Explain the following terms for bearings: - Sliding contact bearing - Roller contact bearing. | (3) (3) |
|  |  | ……4/- |