## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Mechanical Engg./P

Subject: Tool Engineering-I[Cutting Tools](5033)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is the cause of built-up-edge formation on the chip? |  |
| b) | State and explain Taylor’s Tool life equation. |  |
| c) | Explain the following types of tool wear: - Chemical wear - Electro-chemical wear. |  |
| d) | How does tool geometry influence machineability? |  |
| e) | Explain the significance of nose radius of a single point cutting tool. |  |
| f) | State the advantages of H.S.S. as a cutting tool material. |  |
| g) | How are cutting fluids classified? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Show the forces acting on a single point cutting tool giving a neat sketch. |  |
| b) | Distinguish between orthogonal cutting and oblique cutting. |  |
| c) | A pipe is turned on end in orthogonal cutting condition with a tool of 20 rake angle. A chip length of 85 mm is obtained from an uncut chip length of 202 mm, while cutting with a depth of cut of 0.5 mm. Determine the shear plane angle and the chip thickness. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Define machineability. Explain different criteria for machineability. |  |
| b) | Comment on the effects of following machining parameters on tool life: i)Tool material ii)Work material iii)Cutting speed. |  |
| c) | A mild steel bar is turned with H.S.S tool. Determine the tool life for cutting velocity of 40 m/min, if tool life equation is VT 0.2 = C. Also determine cutting speed for a tool life of 60 min. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat labelled sketch of twist drill and state its design features. |  |
| b) | State the functions of clearance angle and rake angle on a cutting tool. |  |
| c) | State the desirable characteristics of a cutting tool material. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are coated carbides? Compare them with plain cemented carbides. |  |
| b) | State the desirable properties of a cutting fluid. |  |
| c) | Comment on the suitability of pure water as a coolant. |  |
|  |  | …..2/- |
|  | -2- |  |
| Q.No.6. | Write notes on any three: | 3 x 4 = 12 |
| a) | Relation between surface finish and chip formation |  |
| b) | Geometry of tool wear |  |
| c) | Tool signature |  |
| d) | Diamond Tools |  |
| e) | Handling of cutting fluids |  |

***********