## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Welding Technology-II(4240)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Compare argon and helium as shielding gases. |  |
| b) | Differentiate between submerged arc welding and electro slag welding. |  |
| c) | State the reasons for using welding fixtures in fabrication shop. |  |
| d) | State the requirements of a good electrode holder. |  |
| e) | Write a note on under water welding. |  |
| f) | Write a note on Weldability test. |  |
| g) | What are the principles governing design of good welding jigs and fixtures? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with a neat sketch MIG welding and state its applications. |  |
| b) | Explain in brief difference between flash and upset welding in resistance welding. |  |
| c) | Explain in brief basic principle of resistance seam welding. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of sectional view explain the working principle of a low pressure welding torch. |  |
| b) | What is meant by residual stresses and how they are introduced in the weld metal? |  |
| c) | Discus in brief the factors affecting distortion in welding. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain in brief electroslag welding by consumable guide tube system. |  |
| b) | Discuss in brief the factors influencing the weld cracking. |  |
| c) | With the help of neat sketch, explain TIG welding and state its applications. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain in brief the factors influencing the procedure of maintenance in welding. |  |
| b) | Explain in brief difference between hot cracking tests and cold cracking tests. |  |
| c) | List modern techniques in welding practices and compare plasma arc welding and laser welding. |  |
| Q.No.6. | Write short notes on any four: | 4 x 3 = 12 |
| a) | Post welding operations |  |
| b) | Electrons beam welding |  |
| c) | Causes and prevention of weld porosity |  |
| d) | Holding and handling of work pieces in FCAW |  |
| e) | Shielding gas mixtures |  |
| f) | Effect of welding position on joint design |  |

***********