## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Production/Automobile Engg.

Subject: Machine Tools & Production Process-II(4060)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | How are shapers classified? State the general precautions while working on a shaper. |  |
| b) | Explain briefly with a diagram the principle on which a planer works. |  |
| c) | What are the essential features common to most milling machines? |  |
| d) | What are the functions of a dividing head? |  |
| e) | How can tapping be done on a drilling machine? |  |
| f) | What is a boring bar? Explain any one method of supporting a boring bar. |  |
| g) | Compare a shaper and a slotter. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State the methods used to obtain Quick return stroke and describe any one of them. |  |
| b) | State and explain with neat sketches the various operations that can be done on a shaper. |  |
| c) | A clear hole of 15mm diameter and 60 mm depth is to be drilled in a job using following particulars: - Spindle feed : 750rpm - Feed : 0.1mm/rev - Approach length : 6.0 mm - Over travel : 4.0mm. Find the time required for drilling Operation. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Under which circumstances is a radial drill used? Sketch a radial drill and label its parts. |  |
| b) | Compare boring operation with reaming. Also sketch and explain counter boring & trepanning operations. |  |
| c) | With a neat labelled sketch, explain the various parts of a Jig boring machine. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the working of a slotted mechanism for driving ram of a slotter. |  |
| b) | How are planning machines classified? Explain the terms: - Clapper box - Cross-rail. |  |
| c) | Why is the bed of a planer twice as long as its table? State the advantages of a pit type planer. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Name the attachments used on a milling machine and state the uses of any three of them. |  |
| b) | Why are milling feeds higher than those for turning? Explain the terms straddle milling and gang milling. |  |
| c) | Sketch and describe the construction of a double column planer. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | “Cutting Speed” defined for the purpose of milling |  |
| b) | Cam milling |  |
| c) | Jobs suitable for machining on vertical milling machine |  |
| d) | Cutting key way in shafts with milling machine |  |
| e) | Arbor assembly |  |

***********