## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Civil Engg./CT/MNE

Subject: Surveying-II(4008)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any three: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | Write down uses of contour plan. |  |
| b) | With a neat figure explain reading the Vernier Theodolite. |  |
| c) | What is orientation of plane table? Explain any one method of orientation. |  |
| d) | Name the accessories of plane table & write down the functions of each. |  |
| Q.No.2. | Answer any two: | 2 x 6 = 12 |
| a) | Name the indirect method of contouring and explain any one with neat figure. |  |
| b) | Explain Re-iteration method of measuring horizontal angle. |  |
| c) | Write down methods of prolonging straight line & explain with a neat figure Double Transiting method. |  |
| Q.No.3. | Answer any two: | 2 x 6 = 12 |
| a) | Explain the sources of errors in Theodolite work. |  |
| b) | Explain Gale’s Traverse table & draw the tabular column. |  |
| c) | An incomplete traverse table is obtained as follows: Line; Length (m); Bearing. PQ; 125.5; N30º 15E. QR; 80.25; S40º 30E. RS; 150.75; S60º 30W. SP; ?; ?. Calculate the length & Bearing of ‘SP’. |  |
| Q.No.4. | Answer any two: | 2 x 6 = 12 |
| a) | Explain Intersection method of plane tabling. |  |
| b) | Explain principle of Tacheometric Survey & write down characteristics of tachometer. |  |
| c) | Explain field procedure for determination of Tacheometric constants. |  |
| Q.No.5. | Answer any two: | 2 x 6 = 12 |
| a) | Explain Rankin’s method of setting out curve. |  |
| b) | Two straight lines AC & CB, to be connected by a 4º curve intersect at a chainage of 2760m. The WCBs of AC & CB are 45º 30 & 75º 30 respectively. Calculate all necessary data for setting out the curve by the method of offsets from the long chord. |  |
| c) | Explain preliminary & field location survey for roads. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Characteristics of contour lines |  |
| b) | Consecutive co-ordinates & total co-ordinates |  |
| c) | Transit Rule |  |
| d) | Reconnaissance survey |  |

***********