## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Shipbuilding Engg.

Subject: Ship Resistance & Propulsion(4339)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) Question NO.1 is compulsory. Answer any five from the following

Questions.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Write short note on any five of the following: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Corresponding speed in model testing |  |
| b) | Residuary resistance |  |
| c) | Froude’s Number |  |
| d) | Pitch of a Propeller |  |
| e) | ‘A’ Bracket |  |
| f) | Twin Screw Propulsion |  |
| Q.No.2. | Answer the following: |  |
| a) | Describe the purpose and procedure for testing a ship model in towing tank. How you scale the results for full scale ship? | (08) |
| b) | A ship whose wetted surfaces area is 5150 m2 travels at 15 Knots. Calculate the frictional resistance if Froude’s frictional constant f=0.422 & Power of speed n=1.825. | (04) |
| Q.No.3. | A propeller rotates at 2 rev/s with a speed of advance of 12 Knots and a real slip of 0.30. The torque absorbed by the propeller is 250 KNm and the thrust delivered is 300 KN. Calculate - The pitch of the propeller - The Thrust Power - The delivered Power. | (12) |
| Q.No.4. | Answer the following: |  |
| a) | With neat sketches explain the following propeller terms: - Rake of a propeller - Skew of a propeller - Expanded profile of a propeller - Propeller diameter | (08) |
| b) | What is a stern tube? List down its functions. | (04) |
| Q.No.5. | Answer the following: |  |
| a) | Explain with a diagram the oil cooled stern tube. | (06) |
| b) | Explain the terms: - Thrust Power - Delivered Power | (06) |

………2/-

-2-

| Q.No.6. | A ship of length 116m, breadth 18.70m, draft 7.1m and displacement 8400 tonnes has the following values of effective power. Speed (knots); 15.5; 16.5; 17.5; 18.5; 19.5. PE(KW); 2300; 2860; 3553; 4390; 5425. Determine for a geometrically similar ship of displacement 9300 tonnes the effective power at a speed of 17.4 knots, Given 1. Kinematic Viscosity= 1.115 x 10-6 m2/s. 2.Wetted surface area of ship of length 116m = 2553m2. | (12) |
| --- | --- | --- |
| Q.No.7. | Answer the following: |  |
| a) | What is the function of a gear box in propulsion system? If a propulsion engine having maximum rpm 1800 has to rotate at a speed of 3.2 rev/s, how can it be achieved? | (09) |
| b) b) | List three advantages of ducted propeller. ### OR List three advantages of CPP (Controllable Pitch Propeller). | (03) (03) |

***********