## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Shipbuilding Engg.

Subject: Introduction to Thermodynamics(4334)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Classify the different systems of thermodynamics giving suitable examples. |  |
| b) | What do you understand by an adiabatic process? Represent an adiabatic compression process on a PV & TS charts. |  |
| c) | What is an air standard cycle? Represent a dual cycle on a PV diagram and give the expression for the efficiency of the cycle. |  |
| d) | Distinguish between a two stroke cycle and a four stroke cycle engine. |  |
| e) | State and explain Fouriers law of heat conduction. |  |
| f) | Differentiate between Wet steam and Super heated steam. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Using Boyle’s law and Charles law, derive the characteristic equation for a perfect gas. What is its utility in thermodynamics? |  |
| b) | A quantity of air at an initial condition of 14 bar, 0.08m3 and 450K expands at constant pressure to 0.25m3. Find - The temperature at the end of the process - Mass of the gas. Given R = 0.287 KJ/KgK and KV =0.711 KJ/KgK. Represent the process on the PV-diagram. |  |
| c) | What is absolute zero of temperature? State and explain the Zeroth law of thermodynamics. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Two kg of air at a pressure of 700 KPa occupies a volume of 0.3m3. This air is expanded adiabatically to volume of 1.5 m3. Find i) Final temperature and pressure ii) the work done iii) the heat transfer. Take Kp = 1.0035 KJ/KgK, Kv = 0.7165 KJ/KgK |  |
| b) | Represent the air standard Otto cycle on a PV and TS diagram. Derive an expression for the Otto cycle efficiency. |  |
| c) | The following data and test results refer to a four cylinder, four stroke cycle oil engine: Cylinder Diameter : 10 cm Stroke : 15 cm Indicated mean effective pressure : 650 KPa Speed : 34 rps brake Torque : 180 N.m Determine i) Indicated power of the engine ii) Brake power of the engine. |  |

………2/-

-2-

| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Determine the condition of steam in each of the following cases: - at a pressure of 10 bar and temperature 210C - at a pressure of 8 bar and volume 0.22 m3/Kg - at a pressure of 12 bar, if 2,600 KJ/Kg are required to produce it from water at 0C. |  |
| b) | Find the external work of evaporation and internal latent heat of 1 kg of steam at a pressure of 12 bar when the steam is 0.8 dry. |  |
| c) | Discuss the different type of heat exchangers. Represent the temperature variation graphically. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with a neat sketch the working principle of a vapour compression refrigeration cycle. |  |
| b) | Give the desirable requirements of a good refrigerant. What is the need for eco friendly refrigerants? |  |
| c) | An ammonia refrigerator works between –10 C and 38 C. The vapour entering the compressor is 100% dry and leaves the compressor at 107C. There is no under cooling. Using P-h chart determine the COP of the cycle. Take Kp for vapour equal to 3.00 KJ/KgK. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Formation of steam |  |
| b) | Specific heats |  |
| c) | Classification of IC engines |  |
| d) | Internal energy of steam |  |

***********