## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Electrical Engg.

Subject: Illumination Engineering(5106)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | In which of the lighting sources, stroboscopic effect is observed? Is it different from flickering of lamp? Give your choice & justify it. |  |
| b) | State the law of illumination applicable for measurement of illumination level at the corner of a room in case the lamp is hung from ceiling and is located at 0.5 m distance from a point of the corner when measured along floor level. Also give the statement of the law applicable for such situation. |  |
| c) | Define LOR, DLOR & ULOR. |  |
| d) | State the ratio of illumination level on visual task, work plane, near vicinity & for vicinity. |  |
| e) | State the objectives of flood lighting. |  |
| f) | With reference to flood lighting define - Beam Spread - Waste light factor - Beam factor. |  |
| g) | Give three important features of monument lighting. |  |
| h) | Give three important features of sports lighting. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Define - Day lighting - Artificial lighting - Supplementary lighting. Also state at least six qualities of good lighting. |  |
| b) | State the different types of lighting schemes and explain each of such schemes in brief with an example of each. |  |
| c) | A 4 feet fluorescent lamp is mounted above the front edge of workbench of 1.5m x 1m size. The mounting height is 1.5 m. The transverse intensity of lamp in lower hemisphere is 600 cd. in all directions. Calculate the illumination level at the centre of workbench. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is the significance of polar curves? State their types. A lamp having MSCP of 800 is suspended at a height of 10 m from ceiling. Determine i) Total flux ii) Illumination level just below the lamp. |  |
| b) | Define i) Lumen ii) Lamp efficacy iii) Lux. State the types of reflections. Also state one example of each luminaire material satisfying each of such reflection principles. |  |
| c) | Give at least two special features of - Entrance lighting - Stair case lighting - Corridor lighting. |  |
|  |  | ……2/- |
|  | -2- |  |
| Q.No.4. | Answer any one of the following Questions: | (12) |
| a) | - Give two points of distinction between; - Uniformity ratio & Direct ratio - Room Index & Cavity Index. - State the factors affecting: - Maintenance factor - Utilization factor. 3) State the factors affecting the selection of luminaries for indoor lighting. | (4) (4) (4) |
| b) | An illumination level of 100 lux is required in a hall of 40m x 8m. The lamps are required to be placed at a height of 3 m above the workbench. Assuming utilization factor of 0.6, Lamp efficacy of 14, Candle depreciation as 18% and suitable space to mounting height ratio, estimate number of lamps required as well as wattage of each lamp. Also draw the layout of lamps. |  |
| Q.No.5. | Answer any one of the following Questions: | (12) |
| a) | With reference to street lighting - State the criteria for good street lighting. - Classify the luminaires used in street lighting design. Also state the characteristics of each. Also give at least one advantage of each - State the factors affecting the choice of street light luminaries. | (3) (6) (3) |
| b) | - List out all the steps in street lighting design. - State the factors affecting the selection of mounting heights of street light luminaries. - Classify the streets w.r.t their average illumination level, uniformity ratio & the type of luminaires preferred & permitted. | (3) (3) (6) |
| Q.No.6. | Answer any one of the following Questions: | (12) |
| a) | With reference to flood lighting - State the types of lamps used. - Classify flood light projectors & also state the basis of their classification. - State the different types of location of flood light projectors. - Give the criteria for selection of flood light projectors. | (2) (3) (3) (4) |
| b) | A building frontage 50m x 15m is to be illuminated by flood light projectors situated 25 m away from it. If illumination level is 100 Lux, C.O.U is 0.5, depreciation factor as 1.5 and waste light factor as 1.2, estimate the number and size of each projector used. Also sketch the projectors layout indicating the usual adjustment provided. |  |

***********