## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Shipbuilding Engg.

Subject: Welding in Shipconstruction-II(4337)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | List the various welding positions for plates (Fillet & groove). |  |
| b) | Write note on “leak test”. |  |
| c) | What are the objectives of hard facing? |  |
| d) | What is the purpose of estimating welding cost? |  |
| e) | Which are the three questions required to be answered while welding dissimilar metals? |  |
| f) | What is the use of jigs & fixture in welding? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Which are the different methods used for welding dissimilar metals? Briefly explain each method. |  |
| b) | What are the problems faced while welding aluminium? How they can be overcome? |  |
| c) | Explain the method of metal spraying in maintenance welding. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List & briefly explain the safety precautions to be observed in gas welding. |  |
| b) | With the help sketch explain the procedure of spot welding. |  |
| c) | Calculate the total cost of manual flux shielded metal arc welding using following data: Welding length 12m Welding speed 12 m/hr Operating factor 30% Labour & overhead charges Rs.25/hr Electrode consumption 3.5 kg/mt. Arc voltage 22 volts Arc current 200 Amps E = 0.6 Electricity charges 1.20 per KWhr Electrode price Rs. 200/kg. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List any six weld symbol along with sketch to show how they are represented on the drawing. |  |
| b) | With the help of sketch explain how the root and facebend test is carried out for weld test. |  |
| c) | What are the advantages & limitations of ultrasonic inspection? |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | What is the purpose of “Impact testing”? How many types of tests are conducted? What are the procedural steps? |  |
| b) | With the help of sketch explain the process of MIG/MAG welding. |  |
| c) | Write short note on - Pre-heat treatment - Post-heat treatment. |  |
| Q.No.6. | Write short notes on any four of the following: | 4 x 3 = 12 |
| a) | Welder’s performance qualification |  |
| b) | Safety precautions while Arc welding |  |
| c) | Welding stainless steel |  |
| d) | Supplementary weld symbols |  |
| e) | Cladding of shaft |  |
| f) | Factors involved in welding cost |  |
| g) | Advantages of TIG welding |  |

***********