## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April,2013 Examinations

Programme: Mining Engg.

Subject: Mine Surveying(5396)

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State general requirement of surveying. |  |
| b) | State precautions in methods of correlation. |  |
| c) | Enumerate factor affecting selection of stope surveying method. |  |
| d) | Write formula for area calculation: i) Cone ii) Trapezium iii) Cylinder. |  |
| e) | Working principle of EDM. |  |
| f) | List out plans and sections required for a new mine. |  |
| Q.No.2. | Answer the following Questions: | 2 x 6 = 12 |
| a) a) | Plot the following survey of a field ABCDEFG and calculate its area. ### OR Explain method of computation of area from plotted plan. |  |
| b) | A coal is stored in a square bay measuring 12m square. The depth of coal stock is 3.6 m, the height of retaining wall is 1.5 and coal is sloped at 45 degree above these walls. Calculate the weight of coal in the stock if the coal has a density of 1.2 m3 per tonne. |  |
| Q.No.3. | Answer the following Questions: |  |
| a) | With sketch explain dip strike and apparent dip of a seam. | (4) |
| b) | Two seam of coal are separated by 50 m (vertical) of strata which dips at 20º to the horizontal. Calculate the length of cross measure drift to connect them - If it is level - It is rising1 in 8 towards the dip of seam - If it dips l in 1.5 in the same direction of seam. | (8) …2/- |
|  | -2- |  |
|  | ### OR |  |
| b) | A vertical bore hole passes through a seam of coal which has dip of 45º. If the apparent thickness of seam ascertained from the core is 10 meters, what is true or stratigraphical thickness of seam? | (8) |
| Q.No.4. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Miners dial |  |
| b) | Role of surveyor in mines |  |
| c) | Survey office |  |
| d) | General requirement of mine plan |  |
| Q.No.5. | Answer the following Questions: |  |
| a) | Write in brief on limits of error as per DGMS. | (3) |
| b) | Explain maintenance of direction and inclination of inclined. | (4) |
| c) | Explain survey in highly inclined winze . | (5) |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Explain direct transfer of azimuth down the shaft. |  |
| b) | Explain steps in correlation. |  |
| c) | Explain alignments of drives. |  |
| d) | Write in brief on Total station. |  |

***********