## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April,2013 Examinations

Programme: Automobile Engg.

Subject: Automobile Engines-I(4054)

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is crank case scavenging? |  |
| b) | List some advantages of an overhead cam shaft. |  |
| c) | List some properties of lubricating oil. |  |
| d) | Draw cross section of a piston and label the parts. |  |
| e) | What is common rail fuel injection? |  |
| f) | Describe a dry liner. |  |
| g) | Explain a dual cycle on a P-V diagram. |  |
| Q.No.2. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Explain Valve operating mechanism. |  |
| b) | Explain the vibration damper with diagram. |  |
| c) | What is a Sodium cooled valve? |  |
| Q.No.3. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Draw diagram and explain the absorber type of muffler. |  |
| b) | A three cylinder engine develops 200KW of break power at 3000 rpm. Its mechanical efficiency is 80% & Mean effective pressure is 12 KPa. If bore and stroke are identical, calculate the bore of the engine. Engine is four-stroke engine. |  |
| c) | Explain the scavenging process in a two-stroke engine. |  |
| Q.No.4. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Draw diagram of a T head SI combustion chamber and explain its utilities. |  |
| b) | What is a CI engine pre-combustion chamber? |  |
| c) | What is a continuous petrol injection system? |  |
| Q.No.5. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Draw a circuit diagram for pump circulated water cooling system and explain. |  |
| b) | How does a Thermostat valve works? |  |
| c) | Explain what is air cooling. |  |
| Q.No.6. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Explain a diesel fuel injector. |  |
| b) | List the advantages of a MPFI engine over a carburetor. |  |
| c) | What is a splash lubricating system? |  |

***********