## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April,2013 Examinations

Programme: Electronics Engg./EC/EI

Subject: Electronic Measurements(4137)

# Time Duration:3 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define the following dynamic characteristics of an instrument: - Speed of Response - Fidelity - Lag. |  |
| b) | List three precautions that should be observed when using an ammeter. |  |
| c) | List the two conditions that must be satisfied when balancing an ac bridge. |  |
| d) | Define the following properties of phosphor - Fluorescence - Phosphorescence - Persistence. |  |
| e) | Explain the function of the delay line in the vertical section of a CRO. |  |
| f) | Draw the circuit of an audio frequency signal generator. |  |
| Q.No.2. | Answer any two: | 2 x 6 = 12 |
| a) | Draw the circuit of: - 3 range ammeter - 3 range voltmeter. |  |
| b) | Draw a block diagram of an analog multimeter. Explain the voltmeter section. |  |
| c) | Draw the circuit of a full wave rectifier ac voltmeter. Explain its working. Draw the waveform of the rectified current through the meter movement. |  |
| Q.No.3. | Answer any two: | 2 x 6 = 12 |
| a) | Explain the causes of any 3 types of errors in measurement. |  |
| b) | Draw the circuit of Maxwell’s bridge & derive the expression for unknown inductance. |  |
| c) | Draw the circuit of Kelvin bridge. Give its application. Illustrate how the Kelvin bridge eliminates the effect of the resistance of the connecting lead. |  |
| Q.No.4. | Answer any two: | 2 x 6 = 12 |
| a) | Draw the block diagram of a CRO. Explain the function of the input attenuator, the trigger circuit and the time base generator. |  |
| b) | Draw a schematic of a triggered time base for an oscilloscope. Explain the generation of the sweep output. |  |
| c) | Draw the block diagram of a sampling oscilloscope. Explain with waveforms the operation of the sampling oscilloscope. |  |

………2/-

-2-

| Q.No.5. | Answer any two: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Draw the block diagram of a frequency divider type of signal generator. Explain how different frequency ranges are generated. |  |
| b) | With the help of a block diagram explain the working of a wideband sweep generator. |  |
| c) | Draw the block diagram of a function generator. Explain generation of triangular and sinusoidal wave. |  |
| Q.No.6. | Answer any two: | 2 x 6 = 12 |
| a) | Explain with a diagram the working principle of a dot matrix display. List one application of this display. |  |
| b) | Explain with a diagram the working principle of – - Bar graph display - Seven segment common anode LED display. |  |
| c) | How are Lissajous patterns generated? Describe measurement of frequency and phase using Lissajous patterns. |  |

***********