## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April,2013 Examinations

Programme: Electrical Engg.

Subject: Generation & Transmission(4114)

# Time Duration: 3 Hrs.                                                  Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | List non conventional energy sources used for generating electricity. |  |
| b) | State the functions of surge tank. Where is it located? |  |
| c) | Explain any three advantages of interconnected grid system. |  |
| d) | Differentiate between Pelton turbine & Francis turbine. |  |
| e) | What is strain type insulator? Where they are used? |  |
| f) | What do you mean by transposition? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain cooling system of diesel engine. |  |
| b) | How is tidal power utilized to generate electrical power? |  |
| c) | A power station has maximum demand of 15000KW. The annual load factor is 50% & plant capacity factor is 40%. Determine the reserve capacity of the plant. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe the various conductors used in transmission line. Also state the advantages & disadvantages of each. |  |
| b) | With a neat single line diagram explain gas turbine power plant. |  |
| c) | Fig.1. shows the spacings of a double circuit 3 phase overhead line. The phase sequence is ABC & the line is completely transposed. The conductor radius is 1.3cm. Find the inductance per phase per kilometer. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | 1.What is the effect of line capacitance on charging the line? |  |
|  | 2.Can line capacitance be neglected while calculating performance of short transmission line? Give reason. |  |
| b) | Explain the phenomena of corona. Also state the various methods to reduce its effect. |  |
| c) | List the advantages & disadvantages of HVDC transmission line. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A 15 km long 3 phase transmission line has a resistance of 5.31 /phase, Inductive reactance of 5.54 /phase. The sending end voltage is 11KV & receiving end load is 1200KW. The sending end p.f. is 0.8 lagging. Find receiving end voltage & current. |  |
| b) | A 200Km long 3 phase overhead line has a resistance of 48.7 ohms per phase, Inductive reactance of 80.20 ohms per phase, capacitance (l-n) is 8.42 nF/Km. It supplies a load of 13.5 MW at a voltage of 88KV & p.f of 0.9 lag. Using nominal T method find sending end voltage & current. |  |
| c) | Explain bundling of conductors. List its advantages. | ….2/- |

-2-

| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Write a short note on effect of ice deposition & wind on the sag. |  |
| b) | Write a short note on vibration damper. |  |
| c) | A transmission line has a span of 150m between level supports. The conductor has a non sectional area of 2cm2. The tension in the conductor is 2000Kg. If specific gravity of conductor material is 9.9 gm/cm3 and wind pressure is 1.5 Kg/m length. Calculate the sag. What is the vertical sag? |  |