## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Electronics Engg./EC

Subject: Television Engineering(5135)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Why are synchronising pulses transmitted along with the picture signal? |  |
| b) | In colour Television system, explain frequency interleaving and its applications. |  |
| c) | Explain the terms (i) additive colour mixing (ii) Luminance. |  |
| d) | Sketch and explain complete channel bandwidth for 625 line TV system. |  |
| e) | What is VSB transmission? Why is it used for transmission of TV picture signal? |  |
| f) | Draw the simplified block diagram of a basic monochrome TV transmitter. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat diagram of vidicon camera tube and describe its working. |  |
| b) | Sketch composite video signal for three successive lines and indicate (i) Extreme white level (ii) Blanking level (iii) Pedestal level. |  |
| c) | Explain why channel bandwidth of 7MHz is essential for successful transmission of audio and video signals in 625 line system. Sketch the channel frequency distribution details for any VHF channel, specifying the important frequencies. |  |
| Q.No.3. | Answer the following Questions: | 2 x 6 = 12 |
| a) | Draw cross sectional view of an image orthicon camera tube labelling various parts. Explain how it develops video signal when light from any scene is focused on its face plate. |  |
| b) | Sketch constructional features of TV receiver picture tube, indicating the potentials applied at the various electrodes. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Why is FM chosen for transmission of sound signal in TV system? Why are pre-emphasis and de-emphasis circuits provided at the FM transmitter and receiver? |  |
| b) | What do you understand by image rejection ratio? Explain how by providing an RF amplifier image signal is rejected? |  |
| c) | Explain briefly how sync pulses are separated from composite video signal and processed. Briefly describe how EHT and boosted B+ voltage are developed from horizontal output circuit. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe with a diagram the construction of a CTV camera. |  |
| b) | Describe the constructional details of PIL colour picture tube. |  |

………2/-

-2-

| c) | Write short notes on following: - Automatic Deguassing circuit - Balun. |  |
| --- | --- | --- |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with the help of a neat block diagram NTSC coder. |  |
| b) | What are safety precautions to be observed in TV servicing? |  |
| c) | Sketch the block diagram of PAL D coder. Mention some important features of PAL system and compare it with NTSC. |  |

***********