## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Fabrication of Boilers & Pressure Vessels(5341)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Discuss any one method of compensating for spring back in bending. |  |
| b) | In roll bending, why rolls are crowned? |  |
| c) | Name the methods of bending tubes. What are the factors on which the bending method selected depends on? |  |
| d) | What are the disadvantages of manual spinning? |  |
| e) | Explain the basic principle of stiffening. |  |
| f) | Explain hoop stress in thin cylindrical shell. |  |
| g) | Explain what is meant by piping load on pressure vessel. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain three press operations that can be performed on power press. |  |
| b) | Mention the hazards present in press brakes and discuss the safety aspects of press brakes. |  |
| c) | Compare the mechanical press brake with hydraulic press brake. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | - Explain the characteristics of the roll-up type pinch roll bending machine. - Explain the characteristics of the three rolls (with adjustable bottom rollers) pyramid roll bending machine. |  |
| b) | - Why are wiper dies used in bending of tubes? - Describe the bending press used in bending of tubes. |  |
| c) | i) Write note on ‘Lubrication for spinning’. ii) Explain briefly the IBR code. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is Hooke’s Law? Define Elastic limit, Proportionality limit, Yield point & Ultimate strength. |  |
| b) | - Explain the Ogee swage method of swaging. - What are the various service conditions commonly encountered by pressure vessels? |  |
| c) | Why is the design of the pressure vessel based on hoop stress? |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the Compound Cylindrical shells? |  |
| b) | What causes brittle fracture of pressure vessels? How can this be prevented? |  |
| c) | - Explain the pit falls in interpretation of welding radiography. - What is Fatigue failure of pressure vessel? |  |

………2/-

-2-

| Q.No.6. | Write short notes on any four: | 4 x 3 = 12 |
| --- | --- | --- |
| a) | Tools for bending thin-wall tubes |  |
| b) | Thermal stresses |  |
| c) | Aluminium alloys as Pressure Vessel material |  |
| d) | Ligament efficiency |  |
| e) | Hydro test of Pressure Vessel |  |
| f) | Wind load on Pressure Vessel |  |

***********