## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Architectural Engg.

Subject: Building Services-II(4533)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) Q.No.1 is compulsory. Answer any four from the remaining

Questions.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define thermal conductivity of a material. What is its significance in choosing a material for thermal insulation? |  |
| b) | Explain Acoustics. |  |
| c) | What is meant by Mechanical ventilation? |  |
| d) | How is noise transmitted? List the different ways and explain briefly. |  |
| e) | What are combustible materials? Mention any two. |  |
| Q.No.2. | Answer the following Questions: | 2x7.5=15 |
| a) | Depending upon the location of air conditioning equipment classify the various systems of air conditioning. |  |
| b) | Write short notes on: - Humidification - Exhaust system of ventilation. |  |
| Q.No.3. | Answer the following Questions: | 2x7.5=15 |
| a) | Describe four methods adopted for carrying out thermal insulation of walls. Support your answer with neat sketches. |  |
| b) | List the general principles of thermal insulation. |  |
| Q.No.4. | Answer the following Questions: | 2x7.5=15 |
| a) | What do you understand by “Acoustics”? Broadly explain how optimum acoustical conditions are achieved in a building by giving an example of a lecture hall in a school campus. |  |
| b) | Explain and draw appropriate sketches showing the construction techniques used in insulation of walls from noise. |  |
| Q.No.5. | Answer the following Questions: | 2x7.5=15 |
| a) | Mention the precautionary measures which can be taken to avoid or minimize dangers of fire. |  |
| b) | Explain the rules to be observed while designing of buildings to make them fire resistant and safe for the people. |  |
| Q.No.6. | Answer the following Questions: | 2x7.5=15 |
| a) | Mention any 3 defects mostly noted in an auditorium and how would you rectify it. |  |
| b) | Explain the thermal insulation adopted for roofs, walls and floors using any one type of thermal insulating material. |  |

***********