## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Mechanical Engg.

Subject: Automobile Engineering(5036)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Draw a neat labelled sketch showing layout of the complete transmission system of a 4WD automobile. |  |
| b) | Name the two components of any suspension system and briefly state the function of each. |  |
| c) | Define camber with the help of a neat sketch and state its purpose. |  |
| d) | Distinguish between: - Port injection and throttle-body injection. - Timed and continuous injection. |  |
| e) | State two possible causes for gear slipping out of engagement and state remedies for each. |  |
| f) | How is the compressor of air-conditioning system of an automobile driven? |  |
| g) | What are feeler gages? Explain their use in I.C. engine maintenance. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat sketch of an automobile chassis frame showing its various members. State the functions of the frame. |  |
| b) | With the help of a neat sketch, explain the construction and working of McPherson strut suspension. What are its advantages over the double-wishbone type? |  |
| c) | Draw a neat labelled sketch of the steering linkage for rigid axle suspension. Explain how wheel alignment is done. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State the advantages of MPFI engine over carburetion engine. |  |
| b) | With the help of a neat block diagram, explain the engine management system for electronic fuel injection in an MPFI engine. |  |
| c) | Having only two groups of injection valves simplifies the MPFI system with little loss of engine performance. Explain. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat labelled sketch of a three-forward and one-reverse speed constant mesh gear box, showing power flow in reverse gear. State the advantages of constant mesh gearbox over sliding mesh type. |  |
| b) | Draw a neat labelled diagram of a torque convertor. Distinguish between fluid coupling and torque convertor. |  |
| c) | With the help of a neat sketch, explain the working of an overdrive. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Draw a neat circuit diagram of a battery and coil ignition system of a petrol engine. What is the function of - Ignition coil and - Condenser. |  |
| b) | List the major components of a lead-acid battery. Draw a neat sketch showing the construction of a battery cell, and explain the working of the lead-acid battery. |  |
| c) | With the help of neat sketches, explain the methods of getting dipped beams from head lights. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch, explain the working of thermostatic expansion valve in the air conditioning system of a car. |  |
| b) | What are the provisions under section 115 of the motor vehicle Act 1988? |  |
| c) | With the help of a neat sketch, explain the use of any one of the following special tools: - Puller - Piston ring compressor. |  |

***********