## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Architectural Engg.

Subject: Building Construction-V(4532)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What are ‘dumb waiters’? |  |
| b) | State the use of shoe in a pile foundation. |  |
| c) | List various materials used in making built in furniture. |  |
| d) | List various materials used in making false ceilings. |  |
| e) | List the suitable materials recommended for sound proof partitions. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Mention the objective of partition wall. State the requirements of an ideal partition. |  |
| b) | Draw a neat labelled elevation of brick nogging partition. |  |
| c) | Write a descriptive note on concrete partitions. |  |
| Q.No.3. | Answer any one of the following Questions: | (12) |
| a) | Draw a neat section through a false ceiling of an auditorium. Draw important details. Make suitable assumption. |  |
| b) | - What are the functions of false ceiling? - Draw a section through false ceiling provided under a G.I. Sheet covered lean to roof. Assume suitable span. |  |
| Q.No.4. | Answer the following Questions: |  |
| a) | Draw a section through the basement of a building showing waterproofing details. | (6) |
|  | ### OR |  |
| a) | Draw a detailed plan & section through a grillage foundation. Make suitable assumption. | (6) |
| b) | - Write a brief note on trussed partition. - Differentiate between precast concrete piles & cast in situ concrete piles. | (6) |
| Q.No.5. | Answer any one of the following Questions: | (12) |
| a) | Government Polytechnic Panaji requires for their toilets modern island counter to contain four washbasins. Prepare neat drawings to show all constructional details necessary for execution. |  |
| b) | A Kitchen cabinet mounted with granite top is to be designed for guesthouse in Polytechnic campus. Prepare necessary drawings showing all constructional details. |  |
| Q.No.6. | Answer the following Questions: |  |
| a) | Draw plan of a machine room of an elevator. | (6) |
|  | ### OR |  |
| a) | Draw a cross section through an elevator for four storeyed building. | (6) |
| b) | State the factors to be considered for choice of various materials used for Built in furniture. | (6) |

***********