## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Electronics & Instrumentation Engg.

Subject: Basic Control Engineering(4219)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.I. | Answer any five of the following: | 5 x 3 = 15 |
| --- | --- | --- |
| 1. | Draw rotary type Potentiometer and obtain its transfer function. |  |
| 2. | Briefly explain the ways to achieve speed control in DC machines. |  |
| 3. | If x(s)=1s(s+5), find x(t). |  |
| 4. | Compare open loop systems and closed loop systems. |  |
| 5. | Sketch typical polar plots for (i) G(jw)=1(jwT1+1) ii) G(jw)=1(jwT). |  |
| 6. | Draw time response specification for a second order system and explain (i) delay time and (ii) peak overshoot. |  |
| 7. | What is meant by marginally stable and conditionally stable system? |  |
| Q.II. | Answer any two of the following: | 2 x 6 = 12 |
| 1. | Describe the typical construction of a synchro and using a diagram, explain any one of its application. |  |
| 2. | Explain the effects of feedback on control system. |  |
| 3. | Briefly describe the construction of solenoid and explain the factors affecting its field strength. |  |
| Q.III. | Answer any three of the following: | 3 x 4 = 12 |
| 1. | Draw the block diagram of a basic negative feedback control system and list the expressions for: (i) OLTF (ii) DTG (iii) CLTF (iv) Actuating signal ratio. |  |
| 2. | Explain the working of stepper motor with a neat diagram. |  |
| 3. | Describe the operation of shaft encoder. |  |
| 4. | A system has a pair of complex conjugate poles P1, P2 = -1j5, a single real zero Z1=-2 and gain factor K=10. Comment on stability of the system showing pole-zero plot. |  |
| Q.IV. | Answer any two of the following: | 2 x 6 = 12 |
| 1. | Obtain the transfer function for R-C network shown in fig.4a. |  |
| 2. | Obtain the expression for steady state error for type1 system to step, ramp and parabolic input. |  |
| 3. | For the system shown in fig.4.b, find i) DTG ii) Loop gain iii) Transfer function iv) Value at point a in terms of c. |  |

………2/-

|  | -2- |  |
| --- | --- | --- |
| Q.V. | Answer any two of the following: | 2 x 6 = 12 |
| 1. | Given a second order system K=9, J=1, F=3, h=1 & R=1. Find m(t), natural frequency, damping frequency and time constant. |  |
| 2. | Draw the bode plot for the phase of transfer function G(s)=ss+100. |  |
| 3. | Describe 4 standard signals (test signals) used in the analysis of control systems. Also represent the signals graphically and mathematically. |  |
| Q.VI. | Answer any two of the following: | 2 x 6 = 12 |
| 1. | By using Routh’s criterion for stability, determine the stability of the system represented by the following characteristics equation. 3s4+10s3+5s2+5s+2=0. |  |
| 2. | Sketch the Nyquist plot for G(s)H(s)=1s(1+3s). |  |
| 3. | Sketch the Bode plot for the magnitude of function G(s)=10s100+s. |  |