## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Mechanical Engg./A

Subject: Theory of Machines(4039)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is a kinematic link? State its characteristics. |  |
| b) | What is fluctuation of speed as related to fly wheel? |  |
| c) | What is stability of a governor? |  |
| d) | What is slip in a belt drive? |  |
| e) | What is a brake? Mention any four types of mechanical brakes. |  |
| f) | State any three applications of cams and followers. |  |
| g) | For a spinning disc mounted on a shaft and an external couple applied through the bearings, show the axis of spin, axis of precession and axis of reactive gyroscopic couple. |  |
| Q.No.2. | Answer the following Questions: |  |
| a) | In the mechanism shown in fig.1, the link AB rotates with a uniform angular velocity of 30 rad/s clockwise. The length of the various links are AB = 100mm, BC = 300mm, BD = 150mm, DE = 250mm, EF = 200mm. Determine the velocity and acceleration of E for the given configuration. | (08) |
| b) b) | For the mechanism shown in Fig.2, determine the velocity and acceleration of the slider B by Klein’s construction. Links OA and AB are 150mm and 750mm long respectively and link OA rotates at a uniform speed of 180 r.p.m. counter –clockwise. ### OR A winding drum raises a cage through a height of 120m. The cage has, at first, an acceleration of 1.5 m/s2 until the velocity of 9 m/s is reached. After this, the velocity remains constant until the cage nears the top, when the final retardation is 6 m/s2. Find the time taken for the cage to reach the top. | (04) (04) |
|  |  | ….2/- |
|  | -2- |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A vertical single start square threaded screw of 50mm core diameter and 12.5mm pitch supports a vertical load of 10KN. The axial thrust on the screw is taken by a collar bearing of 90mm outside diameter and 30mm inside diameter. The coefficient of friction for the screw and nut is 0.14 and that for the collar bearing is 0.11. Find the force required at the end of a lever, which is 400mm long, in order to lift the load. Also determine the efficiency of the screw jack. |  |
| b) | A shaft has a number of collars integral with it. The external diameter of collars is 400mm and shaft diameter is 250mm. If the intensity of pressure is 0.35 N/mm2 and coefficient of friction is 0.05, calculate: - Power absorbed in friction when the shaft runs at 105 r.p.m. carrying a load of 150 KN and - Number of collars required. |  |
| c) | With the help of a neat sketch, explain the construction and working of Porter governor. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A flat belt, 8mm thick and 100mm wide, transmits power between two pulleys running at 1600 m/min. The mass of the belt is 0.9 Kg/m length. The angle of lap in the smaller pulley is 160 and the coefficient of friction between the belt and the pulley is 0.3. If the maximum permissible stress in the belt is 2MPa, find - Maximum power transmitted, and - Initial tension in the belt. |  |
| b) | A V-belt drive consists of three V-belts in parallel on grooved pulleys of the same size. The angle of groove is 30 and the coefficient of friction is 0.12. The cross sectional area of each belt is 800mm2 and the permissible safe stress in the material of the belt is 3 MPa. Calculate the power that can be transmitted between two pulleys 400mm in diameter rotating at 960 r.p.m. |  |
| c) | With the help of a neat sketch, explain the construction and working of crank and slotted lever quick return motion mechanism. |  |
|  |  | ……3/- |

-3-

| Q.No.5. | Answer the following Questions: |  |
| --- | --- | --- |
| a) | A cam rotating clock wise at a uniform speed of 500 r.p.m is required to give a roller follower the following motion: - Follower to move outward with SHM through 40mm duri8ng 120 of cam rotation. - Follower to dwell for next 60 of cam rotation. - Follower to return to its starting position with uniform velocity during next 90 of cam rotation. - Followers to dwell for the rest of cam rotation. The minimum radius of the cam is 50mm and the diameter of the roller is 16mm. The line of stroke of the follower passes through the camshaft axis. Draw the profile of the cam and determine the maximum velocity and acceleration of the follower during the outstroke. | (09) |
| b) b) | Three masses m1, m2 and m3 with magnitudes 10kg, 9kg and 16kg revolve in the same plane at radii 100mm, 125mm and 50mm respectively. The angular positions of m2 and m3 are 60 and 135 from m1 respectively. Determine the position and magnitude of mass m4 at radius of 150mm to balance the system. ### OR What are longitudinal, transverse and torsional vibrations? | (03) (03) |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A spring operated double shoe brake is shown in fig.3. The diameter of the wheel is 500mm. The angle of contact is 90 and the coefficient of friction is 0.3. The effort applied at each arm is 5KN. Determine the brake torque on the wheel. |  |
| b) | With the help of a neat sketch, explain the construction and working of Prony brake dynamometer. |  |
| c)(i) | A compound gear train is shown in Fig.4. It consists of compound gear B-C and D-E. All gears are mounted on parallel shafts. The motor shaft is connected to gear and the output shaft to gear F. The number of teeth on gears A, B, C, D, E and F are 24, 56, 30, 80, 32 and 72 respectively. Determine the train value. What is the speed of gear F if the motor shaft rotates at 800 r.p.m.? | (2) |
| (ii) | Draw the profile of two teeth of a rack to mesh with involute gear of module 4mm and pressure angle 20 | (4) …..4/- |