## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Mining Engg.

Subject: Mine Ventilation(4297)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What are the sources of carbon monoxide in underground coal mine? |  |
| b) | Write short notes on atmospheric pressure. |  |
| c) | Give the salient features of ascentional ventilation. |  |
| d) | In what way turbulent flow create inefficient ventilation? |  |
| e) | Explain the terms: (i) Manometric efficiency (ii) Mechanical efficiency of the fan. |  |
| f) | Explain critical pressure of booster fan. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the physical, chemical properties of methane. How is it detected by flame safety lamp? |  |
| b) | Write short notes on (i) Heat stroke (ii) Cooling power. |  |
| c) | - Explain the principle of flame safety lamp operation. - How do you arrive at percentage of carbon monoxide by the detector? |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | - Write short notes on air-crossing. - How do you prevent air leakages in underground coal mines? |  |
| b) | Derive an expression for equivalent resistance of roadways connected in parallel. What is the equivalent resistance of two roadways of resistance ‘R’ connected in parallel? |  |
| c) | A mine consists of two shafts of 375 mts depth. The air temperature of upcast shaft is 35°C & the downcast shaft is 28°C. The average barometric pressure in d.c. Shaft is 750 mm Hg. Calculate the assistance expressed in Horse power. Does the difference in temperature render when quantity of air passing through the shaft is 120m³/sec? |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Three splits in parallel, of similar cross-section and same type of roadway surface are respectively 300m, 600m and 900m long. Calculate the quantity of air which would flow in each if the total quantity is 300m³/min. |  |
| b) | With a neat sketch explain the construction and working of an axial flow fan. |  |
| c) | An un-lined roadway is 3.5m wide, 2.5m high and 300m long. Calculate (i) Resistance of roadway (ii) Equivalent orifice of the roadway (iii) Water gauge required to maintain a flow of 10 m³/sec through it. Assume co-efficient of friction (K) to be 0.0016. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | - Draw a neat sketch showing the ventilation of long heading by reversible method. Explain the features. - Draw a neat sketch showing the pressure drop of a booster fan located in the outbye of intake gallery. |  |
| b) | - What are the precautions to be taken while doing ventilation survey? - What are the steps involved in quantity survey? |  |
| c) | - Give the design considerations of an evasee. (ii) What are the different causes of resistance to air in the roadways? |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | In what way Natural Ventilation play role in mechanical ventilation of a mine? |  |
| b) | Differentiate between central and bound ventilation system. |  |
| c) | Draw the characteristics curves of centrifugal fan & Explain it. |  |
| d) | Differentiate between Rigid & flexible ducts. |  |

***********