## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Food Technology

Subject: Food Engineering-II(4278)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any ten of the following Questions: | 10x1.5=15 |
| --- | --- | --- |
| a) | Define laminar flow of fluid. |  |
| b) | Give examples of types of food materials that cannot be conveyed by screw conveyor. (any three) |  |
| c) | Name three advantages of a belt conveyor. |  |
| d) | Distinguish between filter aids and filter media. |  |
| e) | List different membrane separation techniques. (at least three) |  |
| f) | Name one continuous and one batch filter. |  |
| g) | What is the advantage of open filters? When are they preferred? |  |
| h) | With the help of a diagram, explain what is dispersed phase of an emulsion. |  |
| i) | In mixing, how would you avoid vertex formation? |  |
| j) | Name two mixers used in the confectionery industry. |  |
| k) | Why is a need felt for temperature control in grinding some food materials? |  |
| l) | Draw a simple sketch of a mango pulper. |  |
| m) | Define mesh size of a powder. |  |
| n) | What is two-stage filling of solids? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | - Distinguish between a conveyor and an elevator. - Explain the criteria for selecting the right conveyor in a given food industry. |  |
| b) | - Name and very briefly explain the working of fluid flow measuring instruments. - Give a brief classification of pumps and select the best pump for the following fluids: 1) A viscous fluid 2) A fluid with solid particles 3) Cream. |  |
| c) | Give an account of vacuum producing devices. With the help of a sketch explain the working of anyone of such equipments and its applications. |  |
| Q.No.3. | Answer any four of the following Questions: | 4 x 3 = 12 |
| a) | Give an account of different types or grades used for sorting fruits and vegetables. |  |
| b) | Explain the process of floating and sedimentation giving examples of applications in the food industry. |  |
| c) | With the help of a diagram, explain the working of a leaf filter. State its applications. |  |
| d) | Explain the principle of centrifugation and the different centrifuges used in the food industry. What are basket centrifuges? |  |
| e) | What should be the characteristics of materials suitable for expression processes? Give examples. With the help of a line sketch, explain the working of a Seiher press. |  |
| f) | Explain the process of solvent extraction. | ….2/- |
|  | -2- |  |
| Q.No.4. | Answer any four of the following Questions: | 4 x 3 = 12 |
| a) | Explain the advantages of crystallization as a food separation process, and the principles on which it is based. Name any two crystallization equipment used in the food industry. |  |
| b) | Explain the mechanism of filtration and hence the advantage of using filter aids. Discuss how these are used to obtain the best possible results. |  |
| c) | Explain what is screen analysis and describe a procedure for the same. |  |
| d) | Explain the advantages of filling on volume basis and the drawbacks of the same. |  |
| e) | Explain the process control of filling carbonated drinks in glass bottles. |  |
| f) | Discuss the packaging machines used for filling of solids in flexible and other containers. (Give suitable examples to illustrate your answer) |  |
| Q.No.5. | Answer any four of the following Questions: | 4 x 3 = 12 |
| a) | With the help of suitable sketches explain the mixing currents in propeller mixers. For which food materials are these suitable? |  |
| b) | Draw a sketch of a dry mixer suitable for mixing powders in similar proportions. Explain its working. |  |
| c) | Give an account of different mixers suitable for dry powder mixing such as i) Spice mixing ii) Fortification of flours iii) Blending of flours. Draw line sketches to illustrate your answer. |  |
| d) | Give an account of mixers used in the bakery industry. |  |
| e) | Which are the mixers used in the chocolate industry? State the salient features of these. |  |
| f) | With the help of a diagram explain the principle and process of homogenization. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | - Based on the forces at play, give a classification of the size reduction equipments used in the food industry. Give suitable examples in each group. - Discuss the features of a hammer mill, with the help of a sketch. |  |
| b) | - Draw a diagram of a size reduction system and explain the importance of sieving and recycling at each step. - What equipment would you select for the grinding of spices? |  |
| c) | Explain the equipments used for grinding of cereals into flour, especially with respect to wheat. Explain the terms crushing rolls, size of feed, size of product. What is the numerical relationship between these? |  |

***********