## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Mechanical Engg./E/FT

Subject: Refrigeration & Airconditioning(5034)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of Psychrometric chart, Mollier’s chart & steam tables is

permissible.

| Q.No.1. | Answer any five of the following: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define “Tonne of Refrigeration”. |  |
| b) | Discuss on Green house effect with respect to refrigerant used. |  |
| c) | Draw the T-S and P-h diagram for the “Vapour compression cycle” with superheated vapour before compression. |  |
| d) | Compare Hermetically sealed compressor with open type compressor. |  |
| e) | Define following: i) Wet bulb depression ii)Dew point depression. |  |
| f) | Atmospheric air with dry bulb temperature of 28C and a wet bulb temperature of 17 C is cooled to 13C without changing its moisture content. Find 1) Original relative humidity 2)Final relative humidity. |  |
| Q.No.2. | Answer any two of the following: | 2 x 6 = 12 |
| a) | With a neat sketch explain the working of Electrolux refrigerator. |  |
| b) | Explain cascade system of refrigerator with T-S and P-h diagram. |  |
| c) | In a 15 TR ammonia refrigeration plant, the condensing temperature in 25C and evaporating temperature –10C. The refrigerant ammonia is subcooled by 5C before passing through the throttle valve. The vapour leaving the evaporator is 0.97 dry. Find 1) Coefficient of performance and 2) power required. Use the following properties of ammonia. |  |
| Saturation Temp. C 25; Enthalpy, KJ/Kg; Entropy, KJ/KgK; Specific heat KJ/KgK. Liquid; Vapour; Liquid; Vapour; Liquid; Vapour. 298.9; 1465.84; 1.1242; 5.0391; 4.6; 2.8. -10; 135.37; 1433.05; 0.5443; 5.4770; -; -. |
| Q.No.3. | Answer any two of the following: | 2 x 6 = 12 |
| a) | What is the purpose of a cooling tower? With a neat sketch explain any one type of cooling tower. |  |
| b) | With a neat sketch explain the working of thermostatic expansion valve. |  |
| c) | Explain the terms RSHF and GSHF applied to Air conditioning. |  |

………2/-

-2-

| Q.No.4. | Answer any two of the following: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | The atmospheric air at 30C dry bulb temperature and 75% relative humidity enters a cooling coil at the rate of 200 m3/min. The coil dew point temperature is 14C and by-pass factor of the coil is 0.1. Determine 1.The temperature of air leaving the cooling coil 2.The capacity of the cooling coil in tonnes of refrigeration and in kilowatt 3.The amount of water vapour removed per minute 4.The sensible heat factor for the process. (show all the points on psychrometric chart) |  |
| b) | 800 m3/mm of recirculated air at 22C DBT and 10C dew point temperature is to be mixed with 300 m3/min of fresh air at 30C DBT and 50% RH. Determine the enthalpy, specific volume, humidity ratio and dew point temperature of the mixture. (show all the points on psychrometric chart) |  |
| c) | Sketch and explain any one type of psychrometer used for measurement of WBT and DBT of air. |  |
| Q.No.5. | Answer any two of the following: | 2 x 6 = 12 |
| a) | With a neat sketch explain “Air Washer’. |  |
| b) | Discuss the different air distribution systems used. |  |
| c) | Explain any one method used for duct design. |  |
| Q.No.6. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Explain a storage type of water cooler with a neat sketch. |  |
| b) | Write a note on Domestic refrigerator. |  |
| c) | Explain any two types of duct systems in Air conditioning plant. |  |

***********