## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Automobile Engg.

Subject: Automobile Engines-II(4061)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Sketch a Wankel engine and state its advantages over reciprocating I.C. engine. |  |
| b) | Explain the functions of ECU and reluctor in Ignition system. |  |
| c) | Electronic Indicators use transducers. List the types and justify the most suitable one. |  |
| d) | Draw a properly labelled sketch of an EGR system. |  |
| e) | What is sludge, how it is formed and how it can be prevented? |  |
| f) | Consumption of lubricating oil increases with increase of engine speed. Is it true or false? Justify your answer. |  |
| g) | Explain the types of heating values of fuel and state the difference between them. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch explain the working of the Bi-starter in the Solex carburator. |  |
| b) | How is a Governor different from Flywheel? Explain the working of a Mechanical Governor. |  |
| c) | Explain the importance of clean diesel in the running of automobiles. Describe briefly processes of cleaning of diesel from the tank to the Injectors. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe the L.T magneto Ignition system using a neat diagram. |  |
| b) | Explain i) Hot and cold spark plugs ii) Factors affecting Ignition timing in SI Engines. |  |
| c) | Explain the TAC Ignition system and give its advantages and disadvantages. |  |
| Q.No.4. |  |  |
| a) | Answer any one of the following Questions: | (6) |
|  | i) Explain Hybrid cars and their advantages. ii) Explain working of Hydraulic valve lifter. |  |
| b) | Answer any one of the following Questions: | (6) |
|  | - With neat sketches explain any two types of supercharging of engines. - Show an arrangement of ‘Wastegate’ for Turbocharger and its working. |  |

………2/-

-2-

| Q.No.5. |  |  |
| --- | --- | --- |
| a) | Answer any one of the following Questions: | (6) |
|  | - Describe the Air Injection method of reducing emissions. - Explain the construction and working of a 3-way catalytic converter. |  |
| b) | Answer any one of the following Questions: | (6) |
|  | - With a neat labelled sketch, explain the working of one type of lubricating oil pump. - What are additives? List the additives and their functions, used in lubricating oils. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with a diagram the exhaust gas analysis using NDIRA method. |  |
| b) | Explain the use of Bosch smoke meter to analyze the exhaust gas. |  |
| c) | The following observations were made from the test of a 4 cylinder, two stroke gasoline Engine: Diameter of Bore = 100 mm Stroke = 150mm Speed = 1600 rpm Area of positive loop of Indicator Diagram = 575 sq.mm Area of negative loop of Indicator Diagram = 25 sq.mm Length of Indicator diagram = 55 mm Spring constant 3.5 bar/cm Find the Indicated power. |  |

***********