## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Civil Engg./CT

Subject: Design of Structures-II[RCC](4016)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of IS 456-200 code is permissible.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | List out various loads to be considered in design of structures in general and R.C. structures in particular. |  |
| b) | State various limit states considered in design of RC structures. |  |
| c) | State salient features of working stress method of design. |  |
| d) | Detail the reinforcement for an R.C. beam with break in direction. |  |
| e) | Explain what do you understand by “Structural integrity”. |  |
| f) | Sketch the reinforcement details for columns with offsets. |  |
| g) | With respect to design and detailing is concerned compare one-way and two-way slabs. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Prove that maximum depth of NA for Fe250, Fe415 and Fe500 steel are 0.53d, 0.48d & 0.46d respectively. |  |
| b) | Design singly reinforced beam to carry an udl of 25KN/m at working condition over an effective span of 4.5m. Take M20 concrete and Fe415 steel. |  |
| c) | A single reinforced beam 230mm x 450mm (overall) consists of 4#12 as tensile reinforcement. If the span of beam is 4m, determine the safe point load at middle of span the beam can support at working condition, in addition to its weight. Take M25 concrete and Fe415 steel. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the “Truss analogy” used in providing the shear reinforcement for an RC beam. |  |
| b) | Design shear reinforcement for a beam 230mm x 450mm (overall). The beam is reinforced with 4#16 and subject to factored load of 45KN/m over 5m effective span. Take M20 concrete and Fe415 steel. |  |
| c) | An R.C. beam 250m x 400mm (effective) consists of 5#12mm as tensile reinforcement. If 2#12mm bars are cranked at 45º near support, design the shear reinforcement to resist SF of 90KN at service condition. Take the shear contribution of cranked bars as per IS recommendations. Assume M25 concrete and Fe415 steel. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | An R.C. beam 250mm x 450mm (overall) is provided with 3#16mm as tensile steel and 2# 12mm as compression steel. Determine the ultimate moment of resistance of beam. If M20 and Fe415 steel are used. |  |

………2/-

-2-

| b) | Design the rectangular beam whose dimension is restricted to 230mm x 400mm to carry an udl of 50KN/m at ultimate condition over an effective span of 4.5m. Take M20 concrete and Fe415 steel. |  |
| --- | --- | --- |
| c) | A beam 230mm x 475mm consists of 3#12 on tension side and 2#12mm on compression side. The effective cover of 40mm each is provided for tension and compression steel. Determine the safe udl the beam can support over an effective span of 5m under working condition. |  |
| Q.No.5. | Answer the following Questions: |  |
| a) | Design a slab for a corridor 2.5m wide and 10m long. Slab is supported alround by 230mm wide beam. M20 concrete and Fe415 steel are used. | (5) |
| b) | Design a two-way slab 5m x4m to carry an imposed load of 2.5KN/m2 and finishing load of 1.0KN/m2. Assume one long edge of slab is continuous and other remaining three edges are discontinuous. M25 concrete and Fe415 steel are used. | (7) |
|  | ### OR |  |
| b) | Design a continuous one-way slab having three equal spans 3.25m each provided for a hall. The LLar slab is 2.0KN/m2 and load due to finishing may be taken as 1.0KN/m2. Use M20 concrete and Fe415 steel. | (7) |
| Q.No.6. | Answer any one of the following Questions: |  |
| a) | Design a short column to carry an axial load of 600KN adopting (i) Square section (ii) Rectangular section. Use M25 concrete and Fe415 steel in each case. | (6+6) |
| b) | Design a square footing to carry an axial column load of 750KN. The size of column is 350mm x 350mm and consists of 8#16mm as longitudinal reinforcement. The bearing capacity of soil is 200KN/m2. Take M20 concrete and Fe 415 steel. Apply necessary checks. | (12) |

***********