## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Electronics & Instrumentation Engg./M

Subject: Industrial Hydro-Pneumatic Control(5113)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain the conventions used in representing direction control valves. |  |
| b) | Draw the symbols to represent the following: - 5/3 way valve with single solenoid - Dual pressure valve - Check valve. |  |
| c) | Explain the application of a quick exhaust valve with the help of a simple circuit. |  |
| d) | Draw the sketch if a telescopic cylinder and state its application. |  |
| e) | Compare hydraulic and pneumatic valves and actuators. |  |
| f) | Describe the different type of Seals. |  |
| g) | Highlight the safety aspect involved with using hydraulic accumulators. |  |
| h) | List 3 applications of pneumatic and hydraulic systems each. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Compare Electrical, Hydraulic and Pneumatic systems. (Give at least six differences). |  |
| b) | What is a hydraulics power pack? List the essential components and explain the functions of each component. |  |
| c) | How is pressure regulated in hydraulic systems? Draw a circuit using pressure regulator. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the working of a 2/2 way direction control valve. Draw a circuit to activate a single acting cylinder using 2/2 way valves. |  |
| b) | Select suitable components to control pneumatic motor in the forward and reverse direction and stop in between. Draw the circuit. |  |
| c) | With the help of diagrams explain how 3/2 valve can be used to implement AND gate and OR gate. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a Semi automatic pneumatic circuit with manual start and automatic return. Explain the working. |  |
| b) | Give an example of a meter in circuit and explain the working. |  |
| c) | The piston rod of a cylinder is to advance only if a protective guard has been lowered and operator presses a push button valve. The piston rod is to retract if the push button is released or the guard is raised. Draw & explain the pneumatic circuit. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Name the components used to create a time delay. Draw a diagram to show how they are connected together. Explain how delay is generated. How can we change the delay time? |  |
| b) | Draw a pneumatic circuit to implement the sequence A+B+A-B-. |  |
| c) | Discuss the installation procedure involving pneumatic components. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the working of a Vane type hydraulic pump with the help of a neat sketch. |  |
| b) | Describe the various types of hoses and connectors used in hydraulic systems. |  |
| c) | List the components of a FRL unit and explain the function of each component. |  |

***********