## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Civil Engg./CT

Subject: Soil Mechanics(4012)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Enlist civil Engineering problems where the knowledge of soil mechanics helps. |  |
| b) | Define: - Bulk unit weight - Porosity - Density Index. |  |
| c) | Define Darcy’s law of permeability. |  |
| d) | State the limitations of Direct shear test. |  |
| e) | Define compaction and consolidation. |  |
| f) | Define the following terms: - Bearing capacity of soil - Ultimate and safe bearing capacity. |  |
| g) | Enlist and explain any one type of shear failure of soil. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Sketch grain distribution curves and explain uniformly graded, well graded, gap graded soil details. |  |
| b) | For the following observations made in the laboratory for specific gravity test: - Empty wt. of sp.gravity bottle = 35.0 gms - Empty wt. of sp.gravity bottle + Soil =39.0 gms - Empty wt. of sp.gravity bottle+Soil+water =42 gms - Empty wt. of sp.gravity bottle +water =37 gms. Find the specific gravity. Also state the significance of Sp.gravity of soil. |  |
| c) | Explain the Atterberg limits of consistency with aid of neat sketch. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Define the term permeability of soil and explain how the co-efficient of permeability is calculated by constant head method. |  |
| b) | A falling head permeameter accommodates a soil sample of 10 cm height and 60 cm2 area. The permeability of the soil sample expected is 1.2 x 10-4 cm/sec. It is desired that the head in the stand pipe to fall from 45cm to 15cm in 38 minutes. Find the size of standpipes. |  |
| c) | Explain plasticity chart and A-line diagram. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with a neat sketch the standard proctor test. |  |
| b) | Explain the factors controlling compaction. |  |

………2/-

-2-

| c) | In a proctor compaction test the following observations where recorded - Weight of empty mould =1950 gms - Volume of mould = 945 cc. Sl.No; 1; 2; 3; 4; 5. Moisture % content; 12%; 14%; 16%; 18%; 20%. Wt. of soil + Mould (gms); 3350; 3550; 3650; 3850; 3750. |  |
| --- | --- | --- |
|  | Find the optimum moisture content and maximum dry density. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with aid of neat sketch the triaxial test. |  |
| b) | Explain the methods of soil exploration. |  |
| c) | Sketch and explain: - Isoclinal fold - Fan Fold - Hinge joint. |  |
| Q.No.6. | Write a short note on any four: | 4 x 3 = 12 |
| a) | Cement stabilization |  |
| b) | Plate load test |  |
| c) | Active and passive earth pressure |  |
| d) | Disturbed and undisturbed samples |  |
| e) | A-line diagram |  |
| f) | Weathering agents |  |

***********