## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Piping Fabrication & Installation(5342)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | List the different steps to arrive at the piping Isometric drawing in process piping. |  |
| b) | Explain in short the Extrusion pipe manufacturing process. |  |
| c) | Sketch the different types of welded flanges used in flange joints in piping. |  |
| d) | Explain the term “Pipe schedule number”. What is its significance? |  |
| e) | Explain the importance of corrosion tables in piping material selection. |  |
| f) | Explain the importance of using a union in screwed joint pipelines. Why union is not used in a flanged joint pipeline? |  |
| g) | Explain the process to inspect the quality of a plastic pipe weld joint. |  |
| Q.No.2. | Design a pipe of material ASTM A53 GrB to fabricate a pipeline to conduct a fluid at 725 F, delivery pressure 70 gms/mm2 and discharge 0.02 m3/sec. The total length of the pipeline is 5742 ft and in its length 1 Gate valve, 2 Globe valves, 7 short radius 90 Elbows, 3 Long radius 90 Elbows and 2 Eccentric Reducers are present. (use given data) | (12) |
| Q.No.3. | Answer the following Questions: |  |
| a) | Draw the orthographic views and the developed view of the position of pipe line whose Isometric view is shown in Fig.1. (Draw out-off scale, but dimension it carefully) | (8) |
| b) | Make a bill of materials of the pipeline shown in Fig.1. Q.3.a. Neglect weight of pipes & fittings. | (4) |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with a neat sketch the draw pipe bending process. |  |
| b) | Draw the end profile of a branch pipe of O.D. 40 mm and hole profile of header pipe of O.D. 80 mm. The branch pipe axis makes an angle of 52 with the axis of header pipe. Both the pipe axis lie in the same plane. |  |
| c) | Explain any one welding process to join plastic pipes. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the procedure to conduct Hydro-test on a pipeline. |  |
| b) | What is critical insulation thickness? Explain its significance. List the insulating materials used for hot and cold insulation. (any two of each) |  |
| c) | Mention any three important design consideration to be given importance, while designing a Toxic gas pipeline. |  |
| Q.No.6. | Write notes on any three: | 3 x 4 = 12 |
| a) | Purging during pipe welding |  |
| b) | Piping material selection |  |
| c) | Pipeline supports |  |
| d) | Importance of P & ID’s |  |
| e) | Pipe stampings | ….2/- |

-2-

### DATA

Table 1:- Pressure drops in pipes

| PIPE NB US GPM | Pressure drop (PSI) per 100 ft long Sh 40 pipe |  |  |
| --- | --- | --- | --- |
|  | 4” | 5” | 6” |
| 250 | 6.30 | 4.01 | 2.78 |
| 275 | 6.93 | 4.41 | 3.06 |
| 300 | 7.56 | 4.81 | 3.33 |

Table 2:- Allowable stresses for ASTM A53  GrB

| Temperature of fluid  F | -20 to 650 | 700 | 750 | 775 |
| --- | --- | --- | --- | --- |
| Allowable stresses (PSI) | 15,000 | 14,300 | 12,900 | 11,850 |

Table 3:-  Pipe properties

| Pipe NB (inches) | Wall thickness in inches |  |  |
| --- | --- | --- | --- |
|  | 40 Sch | 80 Sch | 120 Sch |
| 4 | 0.237 | 0.337 | 0.438 |
| 5 | 0.258 | 0.375 | 0.500 |
| 6 | 0.280 | 0.432 | 0.562 |