## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Architectural Engg.

Subject: Estimation & Costing(4529)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five in short: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Mention the names of any three types of estimates. |  |
| b) | What is schedule of rates? |  |
| c) | What is an abstract sheet used for? |  |
| d) | Mention the market rates for any 3 types of skilled construction labour. |  |
| e) | Give 3 purposes why an estimate is prepared. |  |
| f) | Item wise estimate needs measurement of items, why? |  |
| Q.No.2. | Answer any three of the following Questions: | 3 x 6 = 18 |
| a) | Explain any 2 types of estimates. |  |
| b) | Explain the advantages of item-wise estimate. |  |
| c) | Distinguish between Approximate estimate and detailed estimate. |  |
| d) | Which are the factors responsible for fluctuation in an estimate? |  |
| Q.No.3. | Answer any three of the following Questions: | 3 x 6 = 18 |
| a) | Give the name and unit of measurement of any six R.C.C. items of works. |  |
| b) | Write short notes on: - Measurement sheet - Bill of Quantities. |  |
| c) | Write the detail specifications for: - Internal painting - Single brick masonry wall. |  |
| d) | The electrical fittings required for the kitchen and toilet in figure ‘A’ is as given below. Prepare an abstract of an electrical estimate. Kitchen - 1) Switch board near door 2) Another switch board near platform 3) 2 tube light points 4) 1 fan ceiling point 5) 1 - 5 amp plug point each on both boards 6) 2 - 15 amps plug point near platform. Toilet - 1) switch board near door 2) 1 light point near wash basin mirror 3) 1 - 5amp plug on switch board 4) 1 - 15 amps plug for geyser. |  |
| Q.No.4. | Prepare the measurement and abstract sheet for any two of the following as in figure ‘A’: - Toilet tiling works – floor and wall - Foundation (RCC) and columns (RCC) - External plastering and plaster bands on windows. | (12) |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Define Rate analysis. Which are the various aspects considered in the Rate analysis process of any typical item of works. |  |
| b) | Prepare Rate Analysis for PCC laid in 10cm layer in plinth. Mix 1:4:8 (1 Cement : 4 sand : 8 Aggregate). Assume any additional data and rates. |  |
| c) | Prepare Rate Analysis for supply and fixing of Bathroom Hot and cold mixer with head shower. Assume any additional data and rates. |  |