## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Civil Engg./CT

Subject: Quantity Surveying & Costing(4014)

# Time Duration: 4 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State the obligations of the contractor. |  |
| b) | State different methods of approximate estimates for buildings. |  |
| c) | Briefly explain supplementary estimates. |  |
| d) | What are the attributes of an ideal quantity surveyor? |  |
| e) | What is the signification of work charged establishments in estimation? |  |
| f) | State different method of taking out quantities. Briefly explain centre line method of taking out quantities. |  |
| Q.No.2. | Answer any one of the following Questions: | (15) |
| a) | For the figure No.Q.2(a) showing plan and part section of a single storied house, work out quantities of any five items of the following: |  |
|  | 1.Earthwork in excavation |  |
|  | 2.Laterite masonry in superstructure in cement mortar 1:5 |  |
|  | 3.Damp proof course using cement concrete 1:2:4, 50 mm thick |  |
|  | 4.15 mm external cement plaster 1:3 |  |
|  | 5.PCC 1:3:6 in foundation and plinth |  |
|  | 6.Flooring with ceramic tiles. |  |
| b) | Following table shows chainage, distances and reduced levels along centre line of proposed road. Prepare an estimate for road from chainage 25 to 34 for the data shown below : |  |
| Chainage; 25; 26; 27; 28; 29; 30; 31; 32; 33; 34. Distance (m); 130; 160; 190; 220; 250; 280; 310; 340; 370; 400. R.L. (m); 101.7; 102.3; 101.2; 100.5; 99.8; 100.7; 101.9; 102.8; 103.10; 104.2. |
|  | 1.Draw cross section and longitudinal section |  |
|  | 2.Formation width = 12m |  |
|  | 3.Side slopes in banking are 2:1 and in cutting 1.5:1 |  |
|  | 4.Rise in gradient from chainage 25 to 34 at the rate of 1 in 200 with formation level of 101.2 m at chainage 25 |  |
|  | 5.Use market rates for cutting and embankment. |  |
| Q.No.3. | Answer any three of the following Questions: | 3 x 5= 15 |
| a) | State different types of approvals required to be obtained before execution of the work. Briefly explain importance of each. |  |
| b) | State and explain different situations requiring valuation of property. |  |
| c) | Explain the importance of transportation factor in rate analysis. |  |
| d) | Carry out rate analysis for RCC slab with concrete 1:1:2 including cost of shuttering and centering but excluding cost of steel. |  |
|  |  | ….2/- |
|  | -2- |  |
| Q.No.4. | Answer any three of the following Questions: | 3 x 5 = 15 |
| a) | How the comparative statement is prepared? What are its uses? |  |
| b) | Explain percentage rate contract and lump sum contract. |  |
| c) | Explain the terms: (i)Engineer’s Diary (ii)Measurement Book. |  |
| d) | Draft a tender notice for inviting tenders for construction of a market complex at an estimated cost of Rs.50 Lakhs to be completed in period of 10 months. |  |

| Q.No.5. | Write short notes on any three of the following: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | Valuation of a government building |  |
| b) | Contract documents |  |
| c) | Types of tenders |  |
| d) | Types of specification |  |