## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April,2013 Examinations

Programme: Electronics & Instrumentation Engg.

Subject: Analytical Instrumentation(5115)

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | List three applications of radioisotopes as tracers. |  |
| b) | Discuss the importance of humidity measurement in industry. |  |
| c) | Explain two industrial applications of oxygen analysers. |  |
| d) | Differentiate between insitu and sampling methods used in automatic analysers. |  |
| e) | What is the need to maintain log book in Industry? |  |
| f) | State and explain Beer’s law. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the principle of operation, control section and analyser section of any one type of Gas Chromatograph. |  |
| b) | Discuss three industrial applications of PH measurement. |  |
| c) | Explain the working and application of flame ionization detector. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Name two industries where conductivity measurements are used and explain working of conductivity measurement system. |  |
| b) | With the help of a neat diagram explain the working of a combustion type oxygen analyser. |  |
| c) | Explain principle and working of PH measurement system. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch explain the working of a UV Spectrophotometer. |  |
| b) | Explain the working of a non-dispersive IR Spectrophotometer with the help of a neat sketch. |  |
| c) | Compare different types of radiation and their penetration power. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the working of scintillation counter with the help of a neat diagram. |  |
| b) | Explain the working of a liquid level gauge using radiation technique. |  |
| c) | Explain the working and application of moisture measurement system. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Differentiate between Process and Lab Chromatograph. |  |
| b) | Explain the working of a continuous analyser with the help of a neat diagram. |  |
| c) | Explain the working and applications of Discreet Analyser. |  |

***********