## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Civil Engg./CT

Subject: Transportation Engineering-I(4006)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What are the different means of transportation used in India? How do they help in the national development? |  |
| b) | Explain the necessity of providing the following for the roadway i) Camber ii) Gradient iii) Curves. |  |
| c) | Show the cross section of a WBM road showing different components with diversion. |  |
| d) | Explain the following terms: i) Prime coat ii) Tack coat iii) Seal coat |  |
| e) | What protective works do you suggest for hill roads affected due to heavy monsoon rain? |  |
| f) | Mention any three types of culvert and state their suitability. |  |
| g) | What are the types of gauges used in India? Mention any two advantages of having uniformity in gauges. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat cross section of a road (State Highway) in full embankment/in full cutting and mention all the component parts. |  |
| b) | Write a brief note on providing the following on highways: i) Sight distance ii) super elevation |  |
| c) | What are the different methods of soil stabilization and explain any two in brief. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the following in respect of a WBM road construction: i) Specification of materials used ii) Defects in the WBM roads. |  |
| b) | Differentiate between any three of the following: i) Cut back and emulsion ii) Tar and Asphalt iii) Semi-grout and full grout iv) Single coat and Double coat surface dressing. |  |
| c) | Mention any three types of Bituminous surfaces generally used in the construction and explain any one which is most commonly used in practice. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | - State the advantages and disadvantages of concrete roads. - Types of joints used in concrete road pavements. |  |
| b) | Explain the construction and location with neat sketches, the following: i) Well foundation ii) Caissons. |  |
| c) | Draw a layout of a harbour and indicate the components with functions of each. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Differentiate between the following in respect of location and function: - Low level causeway and High level causeway - Abutment and pier. |  |
| b) | What are the functions of bearings in a bridge? Name the types and explain with neat sketches. |  |
| c) | Explain the following terms in case of a bridge structure: i) Scour ii) Afflux iii) Cut water and ease water iv) Freeboard v) Economic span vi) Linear water way. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Maintenance of Bituminous roads |  |
| b) | Surface drainage on plain and hilly roads |  |
| c) | Factors affecting selection of site for a bridge |  |
| d) | Advantages and shapes of Tunnel |  |
| e) | Maintenance of bridges and culverts |  |

***********