## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Electronics Engg./EC

Subject: Power Electronics(4143)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Give two points of distinction between GTO & LASCR. |  |
| b) | State the precaution to be taken during series operation of diodes. Also draw a neat circuit diagram indicating its operation. |  |
| c) | State any two merits & one demerit of saturable core reactor. |  |
| d) | State the different ratings of a thyrister. |  |
| e) | What is meant by natural commutations? For which thyristor is it applicable? |  |
| f) | Draw the neat circuit diagram of a TRIAC to be used as a static switch. Also state two advantages of such switch. |  |
| g) | What is the significance of free wheeling diode? For which operating load it is used? |  |
| h) | State the two merits & one demerit of servo based regulator. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw Volt Ampere characteristic of SCR and indicate on it the following: 1) VFBO 2) IL 3) IH 4) VRBR. Also define latching current, holding current & state the relationship between them. |  |
| b) | Draw a two transistor model of SCR and state the expression for anode current and indicate clearly each term used in such expression. Can a thyristor continue its conduction, even after gate signal is removed? Give your choice and State clearly the condition for its possibility, if any. Also give reason for your choice. |  |
| c) | State the methods used for triggering of thyristor. Which is the most appropriate method used practically. Give reason for your choice. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw volt ampere characteristic of UJT and explain it in brief. |  |
| b) | Draw the electrical characteristic of TRIAC and explain it in brief. Give 4 applications of TRIAC. What is the function of DIAC in a TRIAC? |  |
| c) | Draw transient characteristics of power diode and explain it in brief. |  |
| d) | Classify the diodes on the basis of their reverse recovery time. Also give at least two important features of each of such diodes. |  |

………2/-

-2-

| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | With reference to single phase bridge controlled rectifier, state the expression for: - Average output voltage - Average output current - R.M.S value of output voltage - R.M.S value of output current Also indicate each term used in such expression. |  |
| b) | Draw a neat circuit diagram of a single phase half wave controlled rectifier operating RL load and draw its supply voltage, output voltage and output current waveform. Also explain the circuit in brief. |  |
| c) | With reference to 3 phase full wave bridge controlled rectifier, state the expression for its output voltage and load current, indicating clearly each term used in such expressions. Also draw its neat circuit diagram & explain its working in brief. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of neat circuit diagram explain the working of MC Murray Inverter. |  |
| b) | Draw a neat circuit diagram of single thyristor chopper and explain its working in brief. |  |
| c) | Draw a neat circuit diagram of single phase cycloconverter used to generate 25 Hz as output frequency for 50 Hz input frequency. Also draw its input voltage, output voltage and output current waveform. Also explain the circuit in brief. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of neat circuit diagram, explain the starting of single phase induction motor using TRIAC. |  |
| b) | State the methods used in power control. Explain any one of such method of your choice with the help of neat circuit diagram used for such control. |  |
| c) | With the help of neat circuit diagram explain the working of saturable core reactor. |  |
| d) | For a relay regulator draw its power as well as control circuit and explain the working of each circuit in brief. |  |

***********