## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Mechanical Engg.

Subject: Tool Engineering-I[Cutting Tools](5033)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What do you mean by Machinability? |  |
| b) | State the various types of coolants used in various machining operations. |  |
| c) | What are the factors on which selection of cutting tool material depends? |  |
| d) | Briefly explain importance of chip thickness ratio. |  |
| e) | Why Relief angles on carbide tools are kept small? |  |
| f) | Explain the meaning of ‘crater wear and flank wear’. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain briefly the mechanics of chips formation with a neat sketch. |  |
| b) | Discuss about the influence of different variables in machinability. |  |
| c) | Derive an expression to show the relationship between chip thickness ratio, shear angle and tap angle. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | How does the tool geometry affect the cutting forces while machining? |  |
| b) | Explain briefly the oblique cutting and orthogonal cutting. |  |
| c) | A 50mm diameter component was machined at 500 rpm using a carbide tipped tool. Tool Failure occurred after 20 minutes. When the speed was decreased to 350 rpm the tool failed at 32 minutes. Find the expected tool life at 440 rpm. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Considering that generation of waste is a social and environmental problem, what care should we take while disposing off used coolants? |  |
| b) | Explain effect of nose radius on finish. |  |
| c) | Explain in detail the use of carbon tool steels as tool material. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the use of water emulsion as a cutting fluid. |  |
| b) | Show the various elements of the following with neat sketches: i) Milling cutters ii) Gear hobes. |  |
| c) | What are the various types of high speed steel used in machining operations? |  |
| Q.No.6. | Write short note on any three: | 3 x 4 = 12 |
| a) | Continuous chips |  |
| b) | Diffusion wear |  |
| c) | Storage and disposal of cutting fluid |  |
| d) | Lubricating and cooling action of cutting fluid |  |

***********