## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Shipbuilding Engg.

Subject: Basic Ship Theory-II(4335)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) Q.No.1 is compulsory. Answer any five from the remaining

questions.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Explain any five of the following: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Lost buoyancy method |  |
| b) | Stowage factor and broken stowage |  |
| c) | List due to flooding |  |
| d) | Use of Bonjean curves in launching calculations |  |
| e) | Hogging and sagging stresses |  |
| f) | Precautions to be taken for inclining experiments |  |
| Q.No.2. | Answer the following: |  |
| a) | A box shaped vessel 65m x 10m x 6m is floating upright on an even keel at 4m draught in salt water. GM=0.6m. Calculate the dynamic stability at 20% heel. | (7) |
| b) | With a neat sketch derive the equation to find moment of statical stability using Attwood’s formula. | (5) |
| Q.No.3. | Answer the following: |  |
| a) | A box shaped vessel 65m x 10m x 6m floats in salt water on an even keel at 5 m draught. A forward compartment 6m long and 10m wide extends from the outer bottom to the height of 3.5m and is full of cargo of permeability 25%. Find the new draught if this compartment is now bilged. | (9) |
| b) | With a neat sketch show that increase in the draught due to flooding of mid-ship compartment is equal to the volume of lost buoyancy to the area of intact water plane. | (3) |
| Q.No.4. | Briefly describe the procedure to carry out launching calculations. | (12) |
| Q.No.5. | Answer the following: |  |
| a) | Sketch and describe load line markings. | (8) |
| b) | Describe the procedure to find freeboard for a typical ship. | (4) |
| Q.No.6. | From the section of box shaped vessel (fig-01), find: | (12) |
|  | i) The position of neutral axis. |  |
|  | ii) Moment of inertia of total area of the material about the neutral axis. |  |
|  | iii) The section modulus about the deck and the keel. |  |
| Q.NO.7. | Answer the following: |  |
| a) | With a neat sketch explain the procedure to find the stress in deck and bottom of the vessel when bending moment is known. | (10) |
| b) | How section modulus of the vessel can be increased by keeping the weight of the vessel to minimum? | (2) |